Drainage Design Management System for Windows

Version 6.8.0

# Tutorial # 7

# Developing a Model With Custom Storm Events

KVL Consultants, Inc.

Developing a Model with Custom Storm Events

Table of Contents

No.	Section	Page

1.0	Introduction	1

2.0	Create a Copy of an Existing Project	1

3.0	Modify Project Defaults	2

4.0	Define / Establish Rainfall Distribution Ids	3

5.0	Define / Establish Rainfall Distribution Data	3

6.0	Create a Draft Model	5

7.0	Run the Model	8

No.	Section	Page

Developing a Model with Custom Storm Event

Date Updated: May 7, 2024

Tutorial Time: 25 minutes

# 1.0	Introduction

This tutorial outlines the method for creating a Project with Custom Storm Event. At present, all Custom Storm Events are limited to using S-Graph for the Unit Hydrograph. The procedure for creating a Custom Storm Event project is basically the same as developing a HEC-1 project using S-Graph. The only difference is that the rainfall uses a custom distribution.

In this procedure, it is assumed that the base HEC-1 project had been developed using S-Graph as the Unit Hydrograph. If the Clark Unit Hydrograph had been used, then some modifications would have to be made to the Sub-Basin data.

# 2.0	Create a Copy of an Existing Project

Copy an Existing Project (‘File  Project Management’) to be used for this tutorial. For this example, create a copy of KVLExample6, which is a project that used the S-Graph Unit Hydrograph.

Enter a short alphanumeric name with no spaces to the “To Project” textbox field. (For this tutorial, enter KVLExample6a). The “To Project” field can hold as much 20 characters. Press the ‘Save’ button to save the data and then press the ‘Copy’ button to copy the project. Then press ‘OK’ to close the form.

# 3.0	Modify Project Defaults

On the Select Project form (‘File  Select Project’), select the new project (KVLExample6a). Click the Details tab to make changes on the Title, Location, and Agency information. Check the ‘Custom Storm Event’ checkbox. You will notice that when ‘Custom Storm Event’ is checked, the Rainfall textbox field changes to CUSTOM and cannot be edited. In addition, the Unit Hydrograph is set to S-Graph and Storms are set to Single.  Neither of these fields can be edited. Finally the Return Periods to model disappeared from the form as they are not relevant for this project. When complete, press ‘Save’ and ‘OK’ to close the Select Project form.

# 4.0	Define / Establish Rainfall Distribution Ids

On the Custom Storm Event Distribution Ids form (‘Hydrology  Custom Storm Event  Distribution Ids’), define the Rainfall Distribution ID. Up to 6 distributions can be set up at any one time. Enter ‘15’ as input to the Time Interval (min) textbox field. For Type, select ‘PC’ from the drop down list.  Enter ‘10’ for the Number of Distribution Records. These values will be the same for all the distributions. Modeling more than one rainfall using the Default distribution can be done by entering a precipitation depth value on the Rainfall column and checking the corresponding Use Default Distribution checkbox. For this tutorial, enter the other data as presented below. Press ‘Save’ and ‘OK’ to close the form.

# 5.0	Define / Establish Rainfall Distribution Data

On the Custom Storm Event Distribution form (‘Hydrology  Custom Storm Event  Distribution Data’), the DEFAULT distribution data must be entered in manually. The number of PC or PI records must be the same as established on the Custom Storm Event Distribution ID form. If more than one rainfall has been entered on the Custom Storm Event Distribution ID form, and the Use Default Distribution check box is NOT checked, then it is necessary to enter additional distribution data for the ID. The Distribution ID can be selected by clicking the ‘Distribution’ button and selecting the ID. Press “OK” to close the form.

# 6.0	Create a Draft Model

Before running the model, it is necessary to create a Draft Model (‘Hydrology  Custom Storm Event  Network’). Click “Create Draft” to create the draft model with the new PC/PI records. Note: the working directory has to be established in ‘File Project  Management’.

Click ‘Yes’ to continue.

Click ‘OK’ to close the Draft HEC-1 Model Network Created form.

After viewing the Draft Model file, exit or close the Editor.

# 7.0	Run the Model

On the Run HEC-1 Model – Custom Storm Event form (‘Hydrology  Custom Storm Event  Model’), check the Distribution 2 and Delete Prior Results check boxes.  Click the ‘Run Model’ button to execute the program.

Distributions will be available to model for all IDs that have a rainfall in the Custom Storm Event Distribution ID form. The process is exactly the same as modeling a HEC-1 model.

This ends the tutorial.