Drainage Design Management System for Windows

Version 6.8.0

# Tutorial # 8

# Developing HEC-1 Routing Data Using GIS

KVL Consultants, Inc.

Developing HEC-1 Routing Data Using GIS

Table of Contents

No.	Section	Page

Tutorial # 8	1

Developing HEC-1 Routing Data Using GIS	1

Developing HEC-1 Routing Data Using GIS	1

Date Updated: May 7, 2024	1

Tutorial Time: 30 minutes	1

1.0	Introduction	1

2.0	Creating the Routing Data in GIS	1

3.0	Update the Routing Data in the HEC-1 Model Using “Routing.Shp”	5

4.0	Update the HEC-1 Model Network	6

5.0	Run the Draft HEC-1 Model	9

6.0	Run the Final HEC-1 Model	9

7.0	View Flow Summary	10

8.0	Backup Project	11

## Developing HEC-1 Routing Data Using GIS

## Date Updated: May 7, 2024

## Tutorial Time: 30 minutes

## Introduction

This tutorial describes how a routing shapefile (“Routing.shp”) is created and how the required attribute fields are defined and populated.  The routing shapefile is a polyline feature which connects the drainage outlet of an upstream sub-basin to the drainage outlet of the immediate sub-basin downstream.  The routing paths define the main flow paths for the drainage runoff.  The routing data are read directly and imported from “Routing.shp” table (i.e., “Routing.dbf”).

## Creating the Routing Data in GIS

To create the “Routing.shp” (i.e., the bold yellow green polylines in Figure 1), we need to ensure that the polyline features for the routing flow paths comprise the main flow paths from the upstream sub-basin drainage outlet to the downstream drainage outlet of the receiving sub-basin. These drainage outlets (represented by the point features on the map) are also called flow concentration points.

The routing polylines (shown in Figure 1) are described in the following table:

| No. | RouteID | U/S Drainage Outlet ID | D/S Drainage Outlet ID |
| --- | --- | --- | --- |
| 1 | 010005 | 010005 | 010010 |
| 2 | 010105 | 010105 | 010110 |
| 3 | 010110 | 010110 | 010010 |
| 4 | 010010 | 010010 | 010015 |

Figure 1 – Relevant GIS Datasets for Tutorial #8

2.1	Data Specification for Required Attributes

In GIS, the associated table (“Routing.dbf”) that defines the required attributes of individual route polylines specific to the routing method used are identified and described in the Required Mapping Fields table (Maps  Import Routing Data  Required Map Fields).

In this tutorial, which employs Kinematic Wave method for channel flow routing, the required attribute field names, data types and formats are provided in the said table. Specific to Kinematic Wave method, the table below lists the required attributes to be created and populated in the creation of the “Routing.shp” table:

| Field Name | Type | Description |
| --- | --- | --- |
| BASINID | Character 2 | Major Basin ID (All) |
| ID | Character 6 | Routing ID (All) |
| ROUTETYPE | Character 20 | Routing Type (Kinematic Wave) |
| LENGTH | Numeric 8.1 | Routing length in feet (All) |
| USGE | Numeric 8.2 | Upstream ground elevation (All) |
| DSGE | Numeric 8.2 | Downstream ground elevation (All) |
| MAN | Numeric 5.3 | Manning’s n for channel (All) |
| ROUTESHAPE | Character 10 | Natural, Channel, Pipe, Box or Roadway |
| WIDTH | Numeric 7.2 | Channel bottom width in feet |
| Z | Numeric 5.2 | Channel side slopes (Z Hor:1 Ver) |
| DIA | Numeric 5.2 | Pipe diameter in inches |

2.2	Populating the Required Attribute Data

After the naming convention of attribute fields as well as their respective data types and formats are known, the attribute field data are created following those specifications. As an example and for illustration purposes, Route ID “010005” has the following field attribute data:

| Field Name | Type | Values | Description |
| --- | --- | --- | --- |
| ID | Character 6 | 010005 | The data is a string with a maximum of 6 characters. |
| BASINID | Character 2 | 01 | The data is a string with a maximum of 2 characters |
| ROUTETYPE | Character 20 | Kinematic Wave | The data is a string with a maximum of 20 characters. (Other ROUTETYPE options are Normal Depth and Muskingum-Cunge) |
| LENGTH | Numeric 8.1 | 3168.0 | The data is numeric with 1-decimal point precision. |
| USGE | Numeric 8.2 | 1585.00 | The data is numeric with 2-decimal point precision. |
| DSGE | Numeric 8.2 | 1540.00 | The data is numeric with 2-decimal point precision. |
| MAN | Numeric 5.3 | 0.040 | The data is numeric with 3-point decimal point precision |
| ROUTESHAPE | Character 10 | NATURAL | The data is a string with a maximum of 10 characters. (Other ROUTESHAPE options are: PIPE, BOX, CHANNEL, and ROADWAY). |
| WIDTH | Numeric 7.2 | 150.00 | The data is numeric with 2-decimal point precision. |
| Z | Numeric 5.2 | 2.00 | The data is numeric with 2-decimal point precision. |
| DIA | Numeric 5.2 | - | The data is numeric with 2-decimal point precision. This attribute field was not included because all the ROUTESHAPE used in the model are all NATURAL. |

The completed attribute table for the “Routing.shp” is shown below.

## Update the Routing Data in the HEC-1 Model Using “Routing.Shp”

To use the routing data assembled in GIS, open the Import HEC-1 Routing Data from GIS Shape File form (Maps  Import Routing Data). Click the button located on the right side of the HEC-1 Routing Map textbox and navigate to the location of the “Routing.shp” or the flow routing shapefile that was created.

Once the file has been located and selected, click OK to close the Open dialog box.

On the Import HEC-1 Routing Data from GIS Shape File form, click the Import GIS Data button. Click Yes to continue. Further, click Yes to close the Warning form. Click OK to close the gridded HEC-1 Routing Data form.  Access Hydrology   HEC-1  Routing to open the routing data imported.

To validate if the imported routing data are correct, go through each individual data item by selecting the Route ID. Click OK to close the HEC-1 Routing Data form.

## Update the HEC-1 Model Network

Select Hydrology HEC-1  Network to open the HEC-1 Model Network form.

Since the naming convention used for the flow routing data (Route IDs) have not changed, the original HEC-1 Model Network should be correct. If changes were made on the Route IDs, the HEC-1 Model Network should be updated to reflect the correct Route IDs.

To create a draft model using the updated routing data, click the “Create Draft” button at the bottom of the form. Click Yes to continue.

Click OK to close the Draft HEC-1 Model Network Created form.

After exiting from the previous form, the program shows automatically the Draft “HEC-1 Model” file. Close the Text Editor after viewing file.  Click “OK” to close the HEC-1 Model Network form

## Run the Draft HEC-1 Model

Select Hydrology  HEC-1  Model to access the HEC-1 model.

Check the ‘10 Year’ Return Period and the ‘Delete Prior Results’ option. Uncheck all other return periods and options. Click ‘Save’ to save the entered preferences.

Click ‘Run Model’ to run the Draft HEC-1 Model. If the model runs without issues, it means that the HEC-1 Model Network that was built for the project is fine.

## Run the Final HEC-1 Model

To run the model for all the return periods, click all the textboxes for the six return periods, and then check the “Update HEC-1” and “Delete Prior Results” options. Click “Save” to save the execution settings and run the model by clicking the “Run Model” button.

If the model runs correctly, it means that the HEC-1 network and data are fine. The results can be viewed by clicking ‘Results’. A full output file can be viewed with the ‘Output’ button, selecting the specific output file to view and pressing ‘OK’. The 100-Year Schematic can be viewed by pressing the ‘Schematic’ button. Press ‘OK’ to close the Run HEC-1 Model form.

## View Flow Summary

Select Hydrology  HEC-1  Flow Summary to access the HEC-1 model results.  Click ‘View’ to view other summary items.

## Backup Project

Select File  Project Management to access the Project Management form. Select ‘Backup Project’ as the Action and TUTORIAL#8 as the project (use the adjacent button to select). Click the ‘Backup’ button to create a backup copy of the project. Select the directory where the backup file should go and finally click ‘OK’ after a directory has been chosen. Click “Yes” to confirm.

A pop-up message appears with the text ‘Backup Project complete!’.

This ends this tutorial.