Drainage Design Management System for Windows

Version 6.8.0

# Tutorial # 8

# Creating a Project File To Evaluate the Impact of Land Use Changes

KVL Consultants, Inc.

Creating a Project to Evaluate the Impact of Land Use Changes

Table of Contents

No.	Section	Page

Tutorial # 8	i

Creating a Project File To Evaluate the Impact of Land Use Changes	i

1.0	Introduction	3

2.0	Create a Model Run Description	3

3.0	Model Run Using GIS Data	6

4.0	Model Run Using Manual Data	7

5.0	Land Use Impact Model Run Summary	9

No.	Section	Page

Creating a Project to Evaluate the Impact of Land Use Changes

Date Updated: May 7, 2024

Tutorial Time: 30 minutes

# 1.0	Introduction

This tutorial provides a working example to determine the impact of changes in land use data. There are two methods that can be used. The first method is to have a second Land Use GIS map and the second method is to modify the Land Use manually for a particular sub basin. For this tutorial, the KVLExample2 project will be used. The development of the basic HEC-1 model has been described in other tutorials. This tutorial is to showcase the use of the Land Use Impacts feature of the program (Hydrology  Land Use Impacts). The menu items for the Land Use Impacts are as follows:

# 2.0	Create a Model Run Description

Open the Land Use Impact Model Run Description form (Hydrology  Land Use Impacts  Model Run Descriptions), to create a Model Run Description.

For each model run it is necessary to create a Model Run ID. In this tutorial, there will be two (2) model runs to be made. One run involves the use of a GIS shape file and the second is one that involves the manual modification of the same land use data.

Enter a unique Model Run ID and select from the dropdown list which Land Use Data Option is used (GIS File or Manual). If a model run uses the GIS File, it is necessary to develop the Sub Basin and Land Use shape files. The change in Land Use dataset from existing condition to future developed conditions should effect a change in the hydrologic model results such as the magnitude of flows.

On the Land Use Impact Model Run Description form (Hydrology  Land Use Impacts  Model Run Descriptions), enter the following data:

| No. | Data Field | Entries |
| --- | --- | --- |
| 1 | Model Run ID | TEST01 |
|  | Land Use Option | GIS File |
|  | Land Use Description | Changed Existing Land Use Code 150 (Residential) to Land Use Code 310 (Industrial) |
|  | Sub Basin GIS Files | C:\...\SUBBASINS.SHP |
|  | Land Use GIS Files | C:\...\LANDUSE.SHP |

| No. | Data Field | Entries |
| --- | --- | --- |
| 2 | Model Run ID | TEST02 |
|  | Land Use Option | Manual |
|  | Land Use Description | Same data as GIS except values are entered manually. |

After entering the data, click the ‘OK’ button to close the form.

# 3.0	Model Run Using GIS Data

On the Run Land Use Impact Model form (Hydrology  Land Use Impacts  Run Model), select ‘TEST01’ for the Model Run and check all the checkboxes for return periods (2 Year, 5 Year, 10 Year, 25 Year, 50 Year, and 100 Year) to be modeled. When ‘TEST01’ is selected for the Model Run, the Land Use Option and Description textbox fields are automatically populated with the data entered earlier (i.e., ‘GIS File’, and ‘Changed Existing Land Use Code 150 (Residential) to Land Use Code 310 (Industrial)’).

If it is preferred to have the model runs saved in a folder (other than the folder established in ‘File  Project Paths’), then check ‘Select Custom Folder’ check box. Before running the Land Use Impact Model, it is assumed that the original model has been run for the same selected return periods. Finally, click the ‘Run Model’ button to execute the program for the dataset. Click ‘Yes’ to continue.

If the ‘Select Custom Folder’ checkbox is checked, you need to create a folder (click ‘Make New Folder’ button) for storing model run results or if a folder already exists navigate to the folder.

After the first land use dataset is successfully run and model run results were saved at a preferred folder location, click the ‘OK’ button to close the Run Land Use Impact Model form.

# 4.0	Model Run Using Manual Data

On the Land Use Impacts form (Hydrology  Land Use Impacts  Manual Land Use Change), select the Details tab.

- Select the Sub Basin (start with Sub Basin ID ‘010005’) to have the land use modified.

- Check the ‘Include’ checkbox to include this record in the analysis

- Select the Existing Land Use Code to be modified (select ‘150’)

- Select the Modified Land Use Code (select ‘310’)

- Enter the Area for this land use that you want to change. In this case the entire area is used (Enter ‘0.0241’)

- Enter a Description for the change (‘Redeveloping residential to an industrial center).

- Repeat steps (a) to (f) for all Sub Basins where Land Use Code ‘150’ are modified to Land Use Code ‘310’.

- After going through all the Sub Basins, click ‘OK’ to exit the Land Use Impacts form.

To run the model (‘TEST02’), use the same steps that were used for running ‘TEST01’.

- Open the Run Land Use Impact Model form (Hydrology  Land Use Impacts  Run Model)

- Check all the checkboxes for the Return Period events (2 Year, 5 Year, 10 Year, 25 Year, 50 Year, and 100 Year)

- Make sure that ‘TEST02’ is selected for the Model Run

- Check ‘Select Custom Folder’ check box to save results in a custom folder

- Click the ‘Run Model’ button.

- Click ‘Yes’ to run the Land Use Impact model.

- Select the folder where to save model run results.

- Click ‘OK’ to continue the run. The Select Directory form closes when the execution is finished.

# 5.0	Land Use Impact Model Run Summary

Open the Land Use Impact Flow Summary form (Hydrology  Land Use Impacts  Model Run Summary) to show the Land Use Change Impact.

Alternatively, the same summary results can be accessed from the Run Land Use Impact Model form clicking the ‘Results’ button.

To check the summary results for other return periods, change the Return Period on the Model View form (Hydrology  Land Use Impacts  Model Run SummaryView) using the Selector button. Click ‘OK’ to close the form.

This concludes this tutorial.