Drainage Design Management System for Windows

Version 6.8.0

# Tutorial # 10

# Total Scour Analysis for Bank Protection

KVL Consultants, Inc.

Total Scour Analysis for Bank Protection

Table of Contents

No.	Section	Page

Tutorial # 10	1

Total Scour Analysis for Bank Protection	1

1.0	Problem Statement	1

2.0	Step-by-Step Procedures	2

2.1	Step 1 - Establish a New Project and Defaults Set-up	3

2.2	Step 2 - Prepare the Cross Section Hydraulics	5

2.3	Step 3 - Compute Total Scour	14

2.3.1	Set up Total Scour Basic Data	14

2.3.2	Calculate the Long Term Scour	16

2.3.3	Calculate the General Scour	18

2.3.4	Calculate the Bedform Scour	20

2.3.5	Calculate the Low Flow Scour	21

2.3.6	Calculate the Total Scour	23

2.4	Step 4 - Report and Documentation of Results	24

Total Scour Analysis for Bank Protection

Date Updated: May 7, 2024

Tutorial Time: 40 minutes

# Problem Statement

To estimate the total scour depth for a channel bank protection at a mild or moderate bend (use “Equilibrium Slope for Sediment-Laden Flow” method for long-term scour, use “Lacey” method for general scour including bend) with the following given conditions:

- The Cross Section “STUDYREACHCROSSSECTION”

- Parameters for the Hydraulics and Geometry:

- Design Flow Rate (cfs): 	3200

- Dominant Flow Rate (cfs):	800

- Channel Slope for Design flow (ft/ft): 	0.015

- Channel Slope for Dominant Flow (ft/ft):	0.015

- Channel Manning’s n for Design Flow:	0.035

- LOB Manning’s n for Design Flow:	0.035

- ROB Manning’s n for Design Flow:	0.035

- Channel Manning’s n for Dominant Flow:	0.030

- LOB Manning’s n for Dominant Flow:	0.030

- ROB Manning’s n for Dominant Flow:	0.030

- The geometry (station and elevation) of this cross section:

| Station (X) | Elevation (Y) | Notes |
| --- | --- | --- |
| 100 | 100 |  |
| 106 | 98 |  |
| 156 | 98 | Left Bank Station |
| 166 | 95 |  |
| 191 | 95 |  |
| 201 | 98 | Right Bank Station |
| 251 | 98 |  |
| 257 | 100 |  |

- D50 (mm) for Study Reach:	1.50

- D84 (mm) for Study Reach: 	10.00

- D16 (mm) for Study Reach:	0.50

- There is a grade control structure located at 800 feet downstream of the channel location for bank protection (cross section “STUDYREACHCROSSSECTION”)

- Distance to Pivot Point (ft):	800

- The Cross Section “SUPPLYREACHCROSSSECTION”

- Parameters for the Hydraulics and Geometry:

- Design Flow Rate (cfs): 	3200

- Dominant Flow Rate (cfs):	800

- Channel Slope for Design Flow (ft/ft): 	0.01

- Channel Slope for Dominant Flow (ft/ft):	0.01

- Channel Manning’s n for Design Flow:	0.035

- LOB Manning’s n for Design Flow:	0.035

- ROB Manning’s n for Design Flow:	0.035

- Channel Manning’s n for Dominant Flow:	0.030

- LOB Manning’s n for Dominant Flow:	0.030

- ROB Manning’s n for Dominant Flow:	0.030

- The geometry (station and elevation) of this cross section:

| Station (X) | Elevation (Y) | Notes |
| --- | --- | --- |
| 100 | 100 |  |
| 106 | 98 |  |
| 181 | 98 | Left Bank Station |
| 191 | 95 |  |
| 216 | 95 |  |
| 226 | 98 | Right Bank Station |
| 301 | 98 |  |
| 307 | 100 |  |

- D50 (mm) for Supply Reach:	2.00

- D84 (mm) for Supply Reach:	12.00

- D16 (mm) for Supply Reach:	1.00

# Step-by-Step Procedures

Step 1: 	Establish a New Project and Default Set-up

Step 2: 	Prepare the Cross Section Hydraulics

Step 3: 	Compute Total Scour

Step 3.1: 	Set up Total Scour Basic Data

Step 3.2: 	Calculate the Long Term Scour

Step 3.3: 	Calculate the General Scour

Step 3.4: 	Calculate the Bedform Scour

Step 3.5: 	Calculate the Low Flow Scour

Step 3.6: 	Calculate the Total Scour

Step 4: 	Report and Documentation of Results

## Step 1 -	Establish a New Project and Defaults Set-up

Click the DDMSW icon on the Desktop or Program menu to launch the DDMSW. Click OK to accept the software disclaimer as is shown in the following figure.

After the DDMSW is launched, the Select Project window is automatically opened as is shown in the following figure.

Click the Add button on the Select Project window to start a new project (Or File  New Project  Add).

On the New Project Options form, select River Mechanics checkbox and click the OK button to close the form.

Type in “BANKPROTECTION1” into the Reference textbox. This is the name of this newly created project. The users can choose the name if it does not exist in the DDMSW database.

Type into the Title textbox a brief descriptive title of this project (Optional).

Type into the Location textbox the location of this project (Optional).

Type into the Agency textbox the agency or company name (Optional).

Check River Mechanics Only checkbox for this project.

Type a detailed description of this project into the comment area under the Project Reference frame (Optional).

Set the Modification Date using today’s date by clicking on the Calendar icon.

Click the Save button to save the entered data.

Click the OK button on the Select Project window to close the window, the following figure shows what the window looks like.

Click the OK button on the pop-up message box.

Note: the Project ID “00157” in the above figure is the database records unique read-only identifier in each computer of the project, which is automatically generated by the program when a new project is created. When the users create a new project, the Project ID of this new project will not be the same as the Project ID shown in the above figure.

## Step 2 -	Prepare the Cross Section Hydraulics

- From the menu bar of the main application window, click River Mechanics  Cross Section Hydraulics to open the River Mechanics – Cross Section Hydraulics window.

Click the Add button on the River Mechanics – Cross Section Hydraulics window to activate all the necessary data entry fields.

Select the “Calculate Data” for the Source (“Enter Data” can also be selected for the Source if the hydraulic results for a cross-section are available.)

Type “STUDYREACHCROSSSECTION” into the Cross Section ID textbox

By default, both the Design and Dominant textboxes in the Entire Cross Section frame are checked. If not, please check these two checkboxes.

Type in “3200” and “800” into the Flow Rate (cfs) textboxes for Design and Dominant, respectively

Type in “0.015” and “0.015” into the Slope (ft/ft) textboxes for Design and Dominant, respectively

Type in “0.035” into the Manning’s n Channel textboxes for Design and Dominant, respectively. Use the same value (“0.035”) for the Manning’s n LOB and Manning’s n ROB textboxes.

Check the Total Scour box. This should display the Main Channel (Bedform Scour) frame. Make sure both Design and Dominant are checked. The form should look as below.

After the data entry, click the Save button. The form should look like the following figure. Click the OK button to close the window.

From the menu bar of the main application window, click River Mechanics  Cross Section Geometry to open the River Mechanics – Cross Section Geometry form.

Click the “Magnifying Glass” on the right side of the ID textbox in the Cross Section frame to open the Select Cross Section ID form. Highlight Cross Section ID “STUDYREACHCROSSSECTION” and click OK to close the form.

Click the Add button on the Natural Cross Sections form and type “100” and “100” into the Station (ft) and Elevation (ft) textboxes, respectively.  Click the Save & Add Record button.

Repeat the above step (n) for the rest of pairs of Station and Elevation values provided below.

| Station (X) | Elevation (Y) | Notes |
| --- | --- | --- |
| 106 | 98 |  |
| 156 | 98 | Left Bank Station |
| 166 | 95 |  |
| 191 | 95 |  |
| 201 | 98 | Right Bank Station |
| 251 | 98 |  |
| 257 | 100 |  |

After all the Station and Elevation values are entered, the Natural Cross Sections form should look like the following figure.

Cross-section data can be imported into DDMSW. However, the XY data must be prepared based on DDMSW XY data format before the cross-section is imported. To know the DDMSW XY data format, export the data of an existing cross-section (File  Export Data  select “Crosssections” under “Hydraulics” Section on the Select a Table to Export form.  Click the Export button and on the Select Fields to Export form, select “XLS” as the Export Type. Click Save to save the setting and click Export. On the Save As form, navigate to where you want to save the exported file and click the Save button. Finally, click OK. Once the file is created, the format can be examined. The format could be used as template for creating importable *.XLS files to DDMSW).

To define the Left Bank Station, select “156.00” on the XY grid table and click the “Magnifying Glass” on the right side of the Left Overbank textbox.

To define the Right Bank Station, select “201.00” on the XY grid table and click the “Magnifying Glass” on the right side of the Right Overbank textbox.

Click OK to close the Natural Cross Sections form.

Go back to the main application window and click River Mechanics  Cross Section Hydraulics from the menu bar to open the River Mechanics – Cross Section Hydraulics form. Click the Update button to update the hydraulic parameter listed on the form by performing a hydraulic analysis using the geometric data provided. The textboxes with yellow highlights will be populated with results from the analysis.

After the update, the River Mechanics – Cross Section Hydraulics form should look like the following figure.

- To create the supply reach cross section data for the project, the user can manually enter the dataset using the same procedure [i.e., Step 2, from (a) to (t)].  In this tutorial, since the data have already been created in another project, the “SUPPLYREACHCROSSSECTION” dataset will be imported.

To import the “SUPPLYREACHCROSSSECTION” dataset, open the Import Cross Sections from Another Project form (River Mechanics  Import Cross Sections from Another Project).

On the Import Cross Sections from Another Project form, use the data provided below. Click Import to import the cross section data into the project.  Select Yes to continue, and hit OK to close the form.

- Import Project Reference: 	PROJECTXSECTIONS

- Option: 	Specific Cross section

- Import Cross Section ID:	SUPPLYREACHCROSSSECTION

To check if the cross section data has been successfully imported, open the Natural Cross Sections form (River Mechanics  Cross Section Geometry).

Compare the geometric data of the “SUPPLYREACHCROSSSECTION” on the Natural Cross Sections form against the tabulated data listed below. “SUPPLYREACHCROSSSECTION” can be selected by clicking on the magnifying icon next to Cross Section ID text box. Make necessary data edits or adjustments on the form, if necessary.  Click OK to close the form.

| Station (X) | Elevation (Y) | Notes |
| --- | --- | --- |
| 100 | 100 |  |
| 106 | 98 |  |
| 181 | 98 | Left Bank Station |
| 191 | 95 |  |
| 216 | 95 |  |
| 226 | 98 | Right Bank Station |
| 301 | 98 |  |
| 307 | 100 |  |

To check if the imported data has all the correct Flow Rates (cfs), Slopes (ft/ft), and Manning’s n (Channel, LOB, and ROB) data, open the Cross Section Hydraulics form (River mechanics  Cross Section Hydraulics). To compare, make sure that the Section ID is set to “SUPPLYREACHCROSSSECTION”. Make any changes necessary to match the form below.

- Cross Section ID: 	SUPPLYREACHCROSSSECTION

- Design Flow Rate (cfs): 	3200

- Dominant Flow Rate (cfs):	800

- Design Slope (ft/ft): 	0.010

- Dominant Slope (ft/ft):	0.010

- Design Manning’s n (Channel, LOB, and ROB):	 0.035

- Dominant Manning’s n (Channel, LOB, and ROB):	0.030

If everything checks out, click the Update button to update the hydraulic analysis results.

On the Select Option form, select “This Record” and click OK. Hit Yes to continue.

Click OK to close the River Mechanics – Cross Section Hydraulics form.

- Creating the two cross sections and evaluating their respective hydraulics in Step 2 are essential steps before proceeding to Step 3 of this tutorial.

## Step 3 - Compute Total Scour

#### Set up Total Scour Basic Data

- From the menu bar of the main application window, click River Mechanics  Scour, to open the Total Scour form.

- Click the Add button on the Total Scour form to activate the data entry fields.

- Type “AZRIVER” into the ID textbox.

- Check the checkboxes for Long Term, General, Bedform, and Low Flow (Note: Do not check Local)

- Click the browse button  in the Method column across Long Term checkbox on the Total tab to launch Select Method window and to select the method to use for Long-term scour analysis.

- On the Select Method form, select the “Equilibrium Slope for Sediment-Laden Flow”, and click OK to close the Select Method form.

- Click the browse button  in the method column across General check box on the Total tab to launch Select Method form.

- Select “Lacey” method from the Select Method form to identify the method to use for General scour analysis. Click OK to close the Select Method form.

Click the Save button to save the entered data. The Total Scour form should look like the following figure.

#### Calculate the Long Term Scour

- Select the Long Term tab as shown in the following figure.

Click browse button  beside the Study Reach Cross Section ID to select the cross section ID “STUDYREACHCROSSSECTION”, and click OK to close the Select Cross Section ID form.

Click browse button  beside the Supply Reach Cross Section ID to select the cross section ID “SUPPLYREACHCROSSSECTION”, and click OK to close the Select Cross Section ID form.

Enter the D50 (mm) values “1.5” and “2.0” for Study and Supply, respectively.

Enter the D84 (mm) values “10” and “12” for Study and Supply, respectively.

Enter the D16 (mm) values “0.5” and “1.0” for Study and Supply, respectively.

Enter “800” into Length to Pivot Pt (ft) textbox

Click the Save button to save the entered data. (The form should look like the following figure).

- Click the Update button to start the computation; select “This Record” from the Selection Option window, and click Yes to continue.

After the update, the final result of the long term scour calculation is shown in the following figure.

#### Calculate the General Scour

- Click the General tab to evaluate the General scour.

Click browse  button beside the Cross Section ID to select the cross section ID “STUDYREACHCROSSSECTION”, and click OK to close the Select Cross Section ID window.

Enter “1.5” into the D50 (mm) textbox.

Click the browse  button beside the Bend Factor, Z textbox to open the Select Factor window and select “Moderate Bend”.

Click the Save button to save the entered data.

Click the Update button and select “This Record” from the Selection Option window. Click Yes on the Calculate General Scour form to proceed.

After the update, the final result of the general term scour calculation is shown in the following figure.

#### Calculate the Bedform Scour

In this section, a procedure on how to calculate the bedform scour will be provided.

On the Total Scour form, select the Bedform tab.  The following figure shows what the window looks like before data entry.

- Click browse  button beside the Cross Section ID to select the cross section ID “STUDYREACHCROSSSECTION”, and click OK to close the Select Cross Section ID window.

Click the Save button to save the data just entered.

Click the Update button and select “This Record” from the Selection Option window. Click Yes to continue.

After the update, the final result of the bedform scour calculation is shown in the following figure.

#### Calculate the Low Flow Scour

In this section, a procedure on how to calculate the low-flow scour will be provided.

On the Total Scour form, select the Low Flow tab.  The following figure shows what the window looks like before data entry.

- Enter “100” into the Low Flow Rate (cfs) textbox.

Click browse  button beside the Channel Material to select the channel material data. Choose “Medium Sand” and click OK to exit the Select Channel Material form.

Click the Save button to save the data just entered.

Click the Update button and select “This Record” from the Select Option form. Click OK.

On the Calculate Low Flow Scour form, click Yes to continue.

After the update, the final result of the low flow scour calculation is shown in the following figure.

#### Calculate the Total Scour

On the Total Scour form, select the Total tab.  The following figure shows what the form looks like.

As shown, the analysis results for the total scour which is the combination of the individual scour components previously analyzed are displayed.

## Step 4 - Report and Documentation of Results

In this section, procedures will be given on how to view, print, and export the calculation results for the total scour analysis.

The total scour is the sum of the long term scour, general scour, local scour, bedform scour and low flow scour. In this tutorial, only the following four scours are covered, that is, the long term scour, general scour, bedform scour and low flow scour.

Make sure all the four scours listed above are checked in the Total tab and all the listed scours are updated.

- To view the results on the screen, click the Print … button on the Total Scour – MB: 01 – ID: AZRIVER form, a report will be generated as shown in the following figure.

To print out the results on a printer, click the printer symbol (      ).

To export the results in PDF format or other formats, click the export symbol (      )

The individual scour components results and cross section hydraulics results can also be viewed, printed, and exported by clicking the Print… button under individual component scour menus and Cross Section Hydraulics menu.

This concludes this tutorial for scour evaluation for bank protection involving mild bends.