Drainage Design Management System for Windows

Version 6.8.0

# Tutorial # 11

# Total Scour for Bridge Piers

KVL Consultants, Inc.

Table of Contents

No.	Section	Page

Tutorial # 11	1

Total Scour for Bridge Piers	1

1.0	Problem Statement	1

2.0	Step-by-Step Procedures	4

2.1	Step 1 -  Establish a New Project and Defaults Set-up	4

2.2	Step 2 - Prepare the Cross Section Hydraulics	7

a.	Import the Bridge Cross Section Data	7

b.	Import the Study Reach Cross Section Data	10

a.	Import the Supply Reach Cross Section Data	12

2.3	Step 3 - Calculate Total Scour	16

2.4	Step 4 - Report and Document the Results	26

Total Scour Analysis for Bridge Piers

Date Updated: May 7, 2024

Tutorial Time: 40 minutes

# Problem Statement

To estimate the total scour depth for a bridge pier (use “Equilibrium Slope for Sediment-Laden Flow” method for long-term scour, use “Neil” method for general scour including a moderate bend, and use the local scour at the piers) with the following given conditions:

- The Cross Section “BRIDGECROSSSECTION”

- Parameters for Hydraulics and Geometry:

- Design Flow Rate (cfs): 	3200

- Dominant Flow Rates (cfs):	800

- Channel Slope for Design Flow (ft/ft):	0.015

- Channel Slope for Dominant Flow (ft/ft):	0.015

- Channel Manning’s n for Design Flow:	0.035

- LOB Manning’s n for Design Flow:	0.035

- ROB Manning’s n for Design Flow:	0.035

- Channel Manning’s n for Dominant Flow:	0.030

- LOB Manning’s n for Dominant Flow:	0.030

- ROB Manning’s n for Dominant Flow:	0.030

- The geometric data (station and elevation) of the cross section:

| Station (X) | Elevation (Y) | Notes |
| --- | --- | --- |
| 100 | 100 |  |
| 106 | 98 |  |
| 131 | 98 | Left Bank Station |
| 141 | 95 |  |
| 166 | 95 |  |
| 176 | 98 | Right Bank Station |
| 201 | 98 |  |
| 207 | 100 |  |

- The Cross Section “STUDYREACHCROSSSECTION”

- Parameters for Hydraulics and Geometry:

- Design Flow Rate (cfs): 	3200

- Dominant Flow Rates (cfs):	800

- Channel Slope for Design Flow (ft/ft):	0.015

- Channel Slope for Dominant Flow (ft/ft):	0.015

- Channel Manning’s n for Design Flow:	0.035

- LOB Manning’s n for Design Flow:	0.035

- ROB Manning’s n for Design Flow:	0.035

- Channel Manning’s n for Dominant Flow:	0.030

- LOB Manning’s n for Dominant Flow:	0.030

- ROB Manning’s n for Dominant Flow:	0.030

- Length to Pivot Point (ft):	800

- The geometric data (station and elevation) of the cross section:

| Station (X) | Elevation (Y) | Notes |
| --- | --- | --- |
| 100 | 100 |  |
| 106 | 98 |  |
| 156 | 98 | Left Bank Station |
| 166 | 95 |  |
| 191 | 95 |  |
| 201 | 98 | Right Bank Station |
| 251 | 98 |  |
| 257 | 100 |  |

- The Cross Section “SUPPLYREACHCROSSSECTION”

- Parameters for Hydraulics and Geometry:

- Design Flow Rate (cfs): 	3200

- Dominant Flow Rates (cfs):	800

- Channel Slope for Design Flow (ft/ft):	0.010

- Channel Slope for Dominant Flow (ft/ft):	0.010

- Channel Manning’s n for Design Flow:	0.035

- LOB Manning’s n for Design Flow:	0.035

- ROB Manning’s n for Design Flow:	0.035

- Channel Manning’s n for Dominant Flow:	0.030

- LOB Manning’s n for Dominant Flow:	0.030

- ROB Manning’s n for Dominant Flow:	0.030

- The geometric data (station and elevation) of the cross section:

| Station (X) | Elevation (Y) | Notes |
| --- | --- | --- |
| 100 | 100 |  |
| 106 | 98 |  |
| 181 | 98 | Left Bank Station |
| 191 | 95 |  |
| 216 | 95 |  |
| 226 | 98 | Right Bank Station |
| 301 | 98 |  |
| 307 | 100 |  |

- Parameters for the Long Term Scour :

- D50 (mm) for Study Reach:	1.50

- D84 (mm) for Study Reach:	10.00

- D16 (mm) for Study Reach:	0.50

- D50 (mm) for Supply Reach:	1.50

- D84 (mm) for Supply Reach:	12.00

- D16 (mm) for Supply Reach:	1.00

- Parameters for the General Scour:

- Exponent m:	Coarse Gravel (0.85)

- Bend Factor, z:	Moderate Bend (0.60)

- D50 (mm):	1.50

- Bend Angle (Degrees):	45.00

- Parameters for the Low Flow Scour:

- Low Flow Rate (cfs):	100.00

- Channel Material	Medium Sand

- Parameters for the Local Scour:

- Pier Width, a (ft): 	2.50

- Pier Length, L (ft):	60.00

- Angle of Attack (Degrees):	30.00

- D50 (mm):	 1.50

- D95 (mm):	20.00

- Nose Shape Factor, K1:	1.0  (Round Nose)

- Bed Condition Factor, K3:	1.2  (Medium Dune)

# Step-by-Step Procedures

Step 1: 	Establish a New Project and Defaults Set-up

Step 2: 	Prepare the Cross Section Hydraulics

Step 3: 	Import Cross Section and Hydraulic Data

Step 4:	Calculate Total Scour at Bridge Piers

Step 4.1:	Set up Total Scour Basic Data

Step 4.2: 	Calculate the Long Term Scour

Step 4.3: 	Calculate the General Scour

Step 4.4: 	Calculate the Local Scour

Step 4.5: 	Calculate the Low Flow Scour

Step 4.6: 	Calculate the Total Scour

Step 5: 	Report and Document the Results

## Step 1 - 	Establish a New Project and Defaults Set-up

- Click the DDMSW icon on the Desktop or Program menu to launch the DDMSW. Click the OK button to accept the software disclaimer as shown in the following figure.

After the DDMSW is launched, the Select Project window is automatically opened as shown in the following figure.

- Click the Add button on the Select Project window to start a new project (Or File  New Project  Add).

- Select River Mechanics checkbox and click the OK button on the New Project Options form.

- Type “BRIDGEPIERFCD1” into the Reference textbox. This is the name of this newly created project. Users can choose any name for the Reference textbox as long as it does not exist in the current DDMSW project database.

- Type into the Title textbox a brief descriptive title for this project. (Optional)

- Type into the Location textbox the location of this project. (Optional)

- Type into the Agency textbox the agency or company name. (Optional)

- Check River Mechanics Only checkbox for this project.

- Type a detailed description of this project into the comment area under the Project Reference frame. (Optional)

- Set the Modification Date using today’s date by clicking on the Calendar icon.

- Click the Save button to save the entered data.

- Click the OK button on the Select Project window and click the OK button on the pop-up message box.  The following figure shows what the window looks like.

Note: the Project ID “00159” in the above figure is the unique database record identifier for the project, which is automatically generated by the program when a new project is created. When users create a new project, the Project ID of the new project will not be the same as the Project ID shown in the above figure.

## Step 2 -	Prepare the Cross Section Hydraulics

All the three (3) cross section data that will be used for this tutorial will be imported from another project.  These cross section data are:

- Bridge Cross Section Data	“BRIDGECROSSSECTION”

- Study Reach Cross Section Data	“STUDYREACHROSSSECTION”

- Supply Reach Cross Section Data	“SUPPLYREACHCROSSSECTION”

#### Import the Bridge Cross Section Data

- To import the first cross section data (Bridge Cross Section Data), open the Import Cross Sections from Another Project form (River Mechanics  Import Cross Sections from Another Project). Use the following data on the form.

- Import Project Reference: 	PROJECTXSECTIONS

- Option: 	Specific Cross section

- Import Cross Section ID:	BRIDGECROSSSECTION

Once the specified data have been selected, click the Import button.  Select Yes to proceed, and hit OK to close the Import Cross Section from Another Project form.

To check if the bridge cross section data has been successfully imported, open the Natural Cross Sections form (River Mechanics  Cross Section Geometry). For the Cross Section ID, select “BRIDGECROSSSECTION” by clicking the Selector button at the right side of the ID textbox.

Compare the geometric data on the Natural Cross Sections form against the tabulated data listed below. Make necessary data edits or adjustments on the form, if necessary.  Click OK to close the form.

| Station (X) | Elevation (Y) | Notes |
| --- | --- | --- |
| 100 | 100 |  |
| 106 | 98 |  |
| 131 | 98 | Left Bank Station |
| 141 | 95 |  |
| 166 | 95 |  |
| 176 | 98 | Right Bank Station |
| 201 | 98 |  |
| 207 | 100 |  |

To check if the imported hydraulic data has all the correct Flow Rates (cfs), Slopes (ft/ft), and Manning’s n (Channel, LOB, and ROB) data, open the Cross Section Hydraulics form (River mechanics  Cross Section Hydraulics). Make sure that the Cross Section ID is set to “BRIDGECROSSSECTION” and compare the data on the form and the following data:

- Cross Section ID: 	BRIDGECROSSSECTION

- Design Flow Rate (cfs): 	3200

- Dominant Flow Rate (cfs):	800

- Design Slope (ft/ft): 	0.015

- Dominant Slope (ft/ft):	0.015

- Design Manning’s n (Channel, LOB, and ROB):	 0.035

- Dominant Manning’s n (Channel, LOB, and ROB):	0.030

If everything checks out, click the Update button to update the hydraulic analysis results. If not, edit the data to match the above figure. On the Select Option form, select “This Record” and click OK. Hit Yes to continue.

Click OK to close the River Mechanics – Cross Section Hydraulics form.

#### Import the Study Reach Cross Section Data

- To import the second cross section data (Study Reach Cross Section Data), open the Import Cross Sections from Another Project form (River Mechanics  Import Cross Sections from Another Project). Use the following data on the form.

- Import Project Reference: 	PROJECTXSECTIONS

- Option: 	Specific Cross section

- Import Cross Section ID:	STUDYREACHCROSSSECTION

Once the specified data have been selected, click the Import button.  Select Yes to continue and hit OK to close the Import Cross Section from Another Project form.

To check if the study reach cross section data has been successfully imported, open the Natural Cross Sections form (River Mechanics  Cross Section Geometry). For the Cross Section ID, select “STUDYREACHROSSSECTION” by clicking the Selector button at the right side of the ID textbox.

Compare the geometric data on the Natural Cross Sections form against the tabulated data listed below. Make necessary data edits or adjustments on the form, if necessary.  Click OK to close the form.

| Station (X) | Elevation (Y) | Notes |
| --- | --- | --- |
| 100 | 100 |  |
| 106 | 98 |  |
| 156 | 98 | Left Bank Station |
| 166 | 95 |  |
| 191 | 95 |  |
| 201 | 98 | Right Bank Station |
| 251 | 98 |  |
| 257 | 100 |  |

To check if the imported hydraulic data has all the correct Flow Rates (cfs), Slopes (ft/ft), and Manning’s n (Channel, LOB, and ROB) data, open the Cross Section Hydraulics form (River mechanics  Cross Section Hydraulics). Make sure that the Cross Section ID is set to “STUDYREACHCROSSSECTION” and compare the data on the form and the following data:

- Cross Section ID: 	STUDYREACHCROSSSECTION

- Design Flow Rate (cfs): 	3200

- Dominant Flow Rate (cfs):	800

- Design Slope (ft/ft): 	0.015

- Dominant Slope (ft/ft):	0.015

- Design Manning’s n (Channel, LOB, and ROB):	 0.035

- Dominant Manning’s n (Channel, LOB, and ROB):	0.030

If everything checks out, click the Update button to update the hydraulic analysis results. If not, edit the data to match the above figure. On the Select Option form, select “This Record” and click OK. Hit Yes to continue.

Click OK to close the River Mechanics – Cross Section Hydraulics form.

#### Import the Supply Reach Cross Section Data

- To import the “SUPPLYREACHCROSSSECTION” dataset, open the Import Cross Sections from Another Project form (River mechanics  Import Cross Sections from Another Project).

On the Import Cross Sections from Another Project form, use the data provided below. Click Import to import the cross section data into the project.  Select Yes to continue and hit OK to close the form.

- Import Project Reference: 	PROJECTXSECTIONS

- Option: 	Specific Cross section

- Import Cross Section ID:	SUPPLYREACHCROSSSECTION

To check if the cross section data has been successfully imported, open the Natural Cross Sections form (River Mechanics  Cross Section Geometry).

Compare the geometric data on the Natural Cross Sections form against the tabulated data listed below. Make necessary data edits or adjustments on the form, if necessary.  Click OK to close the form.

| Station (X) | Elevation (Y) | Notes |
| --- | --- | --- |
| 100 | 100 |  |
| 106 | 98 |  |
| 181 | 98 | Left Bank Station |
| 191 | 95 |  |
| 216 | 95 |  |
| 226 | 98 | Right Bank Station |
| 301 | 98 |  |
| 307 | 100 |  |

To check if the imported data has all the correct Flow Rates (cfs), Slopes (ft/ft), and Manning’s n (Channel, LOB, and ROB) data, open the Cross Section Hydraulics form (River mechanics  Cross Section Hydraulics). To compare, make sure that the Cross Section ID is set to “SUPPLYREACHCROSSSECTION”.

- Cross Section ID: 	SUPPLYREACHCROSSSECTION

- Design Flow Rate (cfs): 	3200

- Dominant Flow Rate (cfs):	800

- Design Slope (ft/ft): 	0.010

- Dominant Slope (ft/ft):	0.010

- Design Manning’s n (Channel, LOB, and ROB):	 0.035

- Dominant Manning’s n (Channel, LOB, and ROB):	0.030

If everything checks out, click the Update button to update the hydraulic analysis results. If not, edit the data to match the above figure.

On the Select Option form, select “This Record” and click OK. Hit Yes to continue.

Click OK to close the River Mechanics – Cross Section Hydraulics form.

- Creating the three cross sections and evaluating their respective hydraulics in Step 2 are essential steps before proceeding to Step 3 of this tutorial.

## Step 3 - Calculate Total Scour

###### Set up Total Scour Basic Data

- From the menu bar of main application window, click River Mechanics  Scour, to open the Total Scour form.

Click the Add button to activate the necessary data entry fields.

Type “PIERNO1” into the ID textbox (this ID indicates that it is for Pier No.1).

Check the checkboxes Long Term, General, Local, and Low Flow (Bed Form is not computed because it will be part of pier local scour computation where the K3 factor, the Bed Condition Factor, will be used).

Click the browse button  in the Method column across Long Term check box to launch long term scour method select menu.

Select the “Equilibrium Slope for Sediment-Laden Flow” from the Select Method window, and click OK to close the Select Method window.

Click the browse button  in the Method column across General check box to launch general scour method select menu.

Select the “Neil and HEC-18” from the Select Method window and click OK to close the Select Method window.

Click the browse button  in the Method column across Local check box to launch local scour method select menu.

Select the “Piers” from the Select Method window and click OK to close the Select Method window.

Click the Save button to save the entered data. The Total Scour – MB: 01 – ID: PIERNO1 window shows up like following figure.

###### Calculate the Long Term Scour

- Click the Long Term tab.

Click browse  button beside the Study Reach Cross Section ID to select the cross section ID “STUDYREACHCROSSSECTION”, and click OK to close the Select Cross Section ID window.

Click browse  button beside the Supply Reach Cross Section ID to select the cross section ID “SUPPLYREACHCROSSSECTION”, and click OK to close the Select Cross Section ID window.

Enter the D50 (mm) values “1.5” and “1.5” for Study and Supply, respectively.

Enter the D84 (mm) values “10” and “12” for Study and Supply, respectively.

Enter the D16 (mm) values “0.5” and “1.0” for Study and Supply, respectively.

Enter “800” into Length to Pivot Pt (ft).

Click the Save button to save the entered data.

Click the Update button to update the data.

Select “This Record” from the Selection Option window and click the Yes button on the Calculate Long Term Scour dialog box to proceed.

After the update, the result of the long term scour calculation shows in the following figure.

Calculate the General Scour

- Click the General tab.

Click the browse  button beside the Bridge Section ID textbox to open the Select Cross Section ID window. Select the “BRIDGECROSSSECTION” and click OK to close the window.

Click the browse  button beside the Upstream Section ID textbox to open the Select Cross Section ID window.  Select the “STUDYREACHCROSSSECTION” and click OK to close the window (Note: Upstream section is for the area upstream of the bridge contraction. It can be generally represented by the study reach cross-section. The supply reach cross-section is not used as the upstream section because it is upstream of the study reach and is generally far away upstream from the bridge).

Click the browse  button beside the Exponent m textbox to open the Select Exponent window.  Select the “Coarse Gravel”, and click OK to close the window.

Click the browse  button beside the Bend Factor, Z textbox to open the Select Factor window. Select the “Moderate Bend” bend factor and click OK to close the window.

Click the browse  button beside the Water Temp (C) textbox to open the Select Temperature window. Select the “20 Degrees Centigrade” and click OK to close the window.

D50 (mm): Use the default value of “1.5” in the textbox (the default value is from the D50 value entered in study reach under Long Term scour menu for Supply). Or enter a value directly in this box. (Note: if a different value is entered here, the D50 value in Long Term for Supply will be changed).

Enter “45” into the Bend Angle (Degrees) textbox

Click the Save button to save the entered data.

Click the Update button on the General tab to update the data.

Select “This Record” from the Selection Option window and click the Yes button on the Calculate General Scour dialog box to proceed.

The following figure shows what the window looks like after the data entry.

###### Calculate the Local Scour

- Click the Local tab.

Click the browse button  beside the Bridge Section ID textbox to open the Select Cross Section ID widow. Select the “BRIDGECROSSSECTION” and click the OK button to close the window.

Enter “2.5” into the Pier Width, (ft) textbox.

Enter “60” into the Pier Length, L (ft) textbox.

Enter “30” into the Angle of Attack (Degree) textbox.

Enter “1.5” into the D50 (mm) textbox.

Enter “20.0” into the D95 (mm) textbox.

Click the browse button  beside the Nose Shape Factor, K1 textbox to open the Nose Shape window. Select “Round Nose” item, and click OK button to close it.

Click the browse button  beside the Bed Condition Factor, K3 textbox to open the Bed Condition window. Select “Medium Dunes” item and click OK button to close the window.

Click the Save button to save the entered data.

Click the Update button to update the data.

Select “This Record” from the Selection Option window and click Yes from the confirmation message to proceed.

After the update the window looks like what is shown in the following figure.

###### Calculate the Low Flow Scour

On the Total Scour form, select the Low Flow tab.  The following figure shows what the window looks like before data entry.

- Enter “100” into the Low Flow Rate (cfs) textbox.

Click browse  button beside the Channel Material to select the channel material data. Choose “Medium Sand” and click OK to exit the Select Channel Material window.

Click the Save button to save the data just entered.

Click the Update button and select “This Major Basin” from the Selection Option window. Click Yes to continue.

After the update the final result of the low flow scour calculation result shows in the following figure

###### Calculate the Total Scour

- Click the Update button to compute the total scour and individual scour components.

Select “This Record” from the Selection Option window to proceed.

After the update the window, the total scour results and individual scour components are displayed as shown in the following figure.

## Step 4 -	Report and Document the Results

In this section, the instruction will be given on how to view, print, and export the calculation results of the total scour.

The total scour is the sum of the long term scour, general scour, local scour, bedform scour and low flow scour. In this tutorial, these scour components are covered.

- To view the results on the screen, click the Print… button on the Total Scour – MB: 01 – ID: PIERNO1 window, a report will be generated as is shown in the following figure.

- To print out the results on a printer, click the printer symbol (      ).

- To export the results in PDF format or other formats, click the export symbol (      )

- The individual scour components results and cross section hydraulics results can also be viewed, printed, and exported by clicking the Print… button under individual component scour menus and Cross Section Hydraulics menu.

This concludes this tutorial for bridge pier scour evaluation.