Drainage Design Management System for Windows

Version 6.8.0

# Tutorial # 13

# Riprap Sizing Calculation

KVL Consultants, Inc.

Riprap Sizing Analysis

Table of Contents

No.	Section	Page

Tutorial # 13	1

Riprap Sizing Calculation	1

Riprap Sizing Calculation for Bank Protection	1

1.0	Problem Statement	1

2.0	Step-by-Step Procedures	1

2.1	Step 1 - Establish a New Project and Defaults Set-up	2

2.2	Step 2 - Prepare the Cross Section and Hydraulic Data	4

2.3	Step 3 - Calculate Riprap Size	8

2.4	Step 4 - Report and Document the Results	10

#### Riprap Sizing Calculation for Bank Protection

Date Updated: May 7, 2024

Tutorial time: 25 minutes

# Problem Statement

To estimate the riprap sizing for bank protection using “Channel Banks on Curved Reach” type with the following given design parameters:

- The Cross Section “STUDYREACHROSSSECTION”

- Parameters for Hydraulics and Geometry:

- Design Flow Rate (cfs): 	3200

- Channel Slope (ft/ft): 	0.015

- Design Manning’s n (Channel, LOB, ROB):	0.035

- The geometric data (station and elevation) of the cross section:

| Station (X) | Elevation (Y) | Notes |
| --- | --- | --- |
| 100 | 100 |  |
| 106 | 98 |  |
| 156 | 98 | Left Bank Station |
| 166 | 95 |  |
| 191 | 95 |  |
| 201 | 98 | Right Bank Station |
| 251 | 98 |  |
| 257 | 100 |  |

- Parameters for Channel Banks:

- Bank Slope Angle (Degrees):	45.00

- Specific Weight for Stone (lb/cu ft):	150.00

- Specific Weight for Water (lb/cu ft):	62.40

# Step-by-Step Procedures

Step 1: 	Establish a New Project and Default Set-up

Step 2: 	Prepare the Cross Section Geometry

Step 3: 	Calculate Riprap Sizing

Step 4: 	Report and Document the Results

## Step 1 -	Establish a New Project and Defaults Set-up

- Click the DDMSW icon on the Desktop or Program menu to launch the DDMSW. Click OK to accept the software disclaimer as is shown in the following figure.

After the DDMSW is launched, the Select Project window is automatically opened as is shown in the following figure.

Click the Add button on the Select Project window to start a new project (or you can start a new project by File  New Project  Add).

Select River Mechanics checkbox and click the OK button on the New Project Options form.

Type “RIPRAPSIZING1” into the Reference textbox. This is the name of this newly created project. Users can choose any name for the Reference textbox as long as it does not exist in the current DDMSW project database.

Type into the Title textbox a brief descriptive title of this project. (Optional)

Type into the Location textbox the location of this project. (Optional)

Type into the Agency textbox the agency or company name. (Optional)

Check River Mechanics checkbox for this project.

Type a detailed description of this project into the comment area under the Project Reference frame. (Optional)

On the Modification Date, use the current date.

Click Save button to save the entered data.

Click OK button on the Select Project window, and click OK button on the pop-up message box.  The following figure shows what the window looks like.

Note: the Project ID 00161 in the above figure is the database records unique read-only identifier of the project, which is automatically generated by the program when a new project is created. When the users create a new project, the Project ID of this new project will not be the same as the Project ID shown in the above figure.

## Step 2 -	Prepare the Cross Section and Hydraulic Data

Only one (1) cross section data, the “STUDYREACHCROSSSECTION”, will be used for this tutorial. This cross section data will be imported from another project.

- To import the first cross section data (Study Location Cross Section Data), open the Import Cross Sections from Another Project form (River Mechanics  Import Cross Sections from Another Project). Use the following data on the form.

- Import Project Reference: 	PROJECTXSECTIONS

- Option: 	Specific Cross section

- Import Cross Section ID:	STUDYREACHCROSSSECTIONS

Once the specified data have been selected, click the Import button.  Select Yes to proceed, and hit OK to close the Import Cross Section from Another Project form.

To check if the bridge cross section data has been successfully imported, open the Natural Cross Sections form (River Mechanics  Cross Section Geometry). For the Cross Section ID, select “STUDYREACHCROSSSECTION” by clicking the Selector button at the right side of the ID textbox.

Compare the geometric data on the Natural Cross Sections form against the tabulated data listed below. Make necessary data edits or adjustments on the form, if necessary.  Click OK to close the form.

| Station (X) | Elevation (Y) | Notes |
| --- | --- | --- |
| 100 | 100 |  |
| 106 | 98 |  |
| 156 | 98 | Left Bank Station |
| 166 | 95 |  |
| 191 | 95 |  |
| 201 | 98 | Right Bank Station |
| 251 | 98 |  |
| 257 | 100 |  |

To check if the imported hydraulic data has all the correct Flow Rates (cfs), Slopes (ft/ft), and Manning’s n (Channel, LOB, and ROB) data, open the Cross Section Hydraulics form (River mechanics  Cross Section Hydraulics). Make sure that the Cross Section ID is set to “STUDYREACHCROSSSECTION” and only the “Design” checkbox is checked. Please note the dominant flow event will not be used in the Riprap Sizing analysis.

Compare the imported data on the form against the actual data as follows:

- Cross Section ID: 	STUDYREACHCROSSSECTION

- Design Flow Rate (cfs): 	3200

- Design Slope (ft/ft): 	0.015

- Design Manning’s n (Channel, LOB, and ROB):	 0.035

If everything checks out, click the Update button to update the hydraulic analysis results. If not, make sure to replicate the above figure, then click Update.

On the Select Option form, select “This Record” and click OK. Hit Yes to continue.

After the update, the River Mechanics – Cross Section Hydraulics window looks like the following figure.

Click OK to close the River Mechanics – Cross Section Hydraulics form.

## Step 3 -	Calculate Riprap Size

- From the menu bar of main application window, click River Mechanics Riprap to open the River Mechanics - Riprap window.

Click Add button on the River Mechanics - Riprap window.

Enter “CHNL1” into the Location ID textbox

Browse for “Channel Banks on Straight Reach” in the Type textbox

Click OK on the Select Type dialog box.

Check Use Cross Section ID check box

Browse for “STUDYREACHCROSSSECTION” in the Section ID textbox.

Click OK on the Select Cross Section ID dialog box.

Enter “3” into the Bank Slope (H:V) textbox

Enter “150.00” into the Specific Weight Stone (lb/cu ft) textbox

Enter “62.43” into the Specific Weight Water (lb/cu ft) textbox

Select “Angular” from the drop down for Riprap Type in the Safety Factor frame.

Click the Save button.

Click Update button to compute riprap median size D50 (ft).

Highlight “This Record” in the Selection Option window and click OK. Click Yes when the Calculate Riprap Size dialog box opens.

After the update process is finished, the window looks like what is shown in the following figure. Click OK to close the window.

## Step 4 -	Report and Document the Results

In this section, the instruction will be given on how to view, print, and export the calculation results of the riprap sizing.

- Click the Print… button on the River Mechanics – Riprap window.  A report will be generated as is shown in the following figure.

To print the results, click the printer symbol (      ).

To export the results in PDF format or other formats, click the export symbol (      )

More detailed information for cross section hydraulics can also be viewed, printed, and exported by clicking the Print… button under Cross Section Hydraulics menu.

This concludes this tutorial for sediment yield analysis.