Drainage Design Management System for Windows

Version 6.8.0

# Tutorial # 14

# Lateral Erosion Analysis

KVL Consultants, Inc.

Lateral Erosion Analysis

Table of Contents

No.	Section	Page

Tutorial # 14	1

Lateral Erosion Analysis	1

Riprap Sizing Analysis for Bank Protection	1

1.0	Problem Statement	1

2.0	Step-by-Step Procedures	3

2.1	Step 1 - Establish a New Project and Defaults Set-up	3

2.2	Step 2 – Prepare the Cross Section and Hydraulics Data	6

2.3	Step 3 - Calculate Lateral Erosion Distance	11

2.4	Step 4 - Report and Documentation of Results	26

#### Riprap Sizing Analysis for Bank Protection

Date Updated: May 7, 2024

Tutorial Time: 40 minutes

# Problem Statement

To estimate the lateral erosion corridor for a watercourse as a basis to protect the public from potential flood encroachments caused by unmitigated lateral bank migration.

The following data are provided for this tutorial:

- The Cross Section “LATEROS1”

- Parameters for Hydraulics and Geometry:

- Design Flow Rate (cfs): 	3200

- Channel Slope (ft/ft): 	0.015

- Design Manning’s n for Main Channel:	0.035

- Design Manning’s n for LOB:	0.045

- Design Manning’s n for ROB:	0.050

- The geometric data (station and elevation) of the cross section:

| Station (X) | Elevation (Y) | Notes |
| --- | --- | --- |
| 0 | 165.0 |  |
| 200 | 164.9 |  |
| 215 | 164.5 | FEMA Floodway Left Station |
| 275 | 164.3 |  |
| 318 | 164.1 | Left Bank Station |
| 320 | 163.9 |  |
| 321 | 163.85 |  |
| 325 | 163.8 |  |
| 340 | 163.6 |  |
| 345 | 160.0 |  |
| 350 | 159.9 |  |
| 355 | 159.6 | Thalweg Station |
| 365 | 159.7 |  |
| 370 | 160.0 |  |
| 375 | 161.0 |  |
| 380 | 163.0 | Right Bank Station |
| 390 | 163.2 |  |
| 420 | 164.0 | FEMA Floodway Right Station |
| 515 | 164.3 |  |
| 600 | 164.6 |  |
| 900 | 165.0 |  |

- Data for Alternative Analyses

- Alternative Scenario #1:  No Data Available

- Channel Depth, D (ft):	4.5

- Alternative Scenario #2:  Scour Data Available

- Channel Depth, D (ft):	4.5

- Scour Depth, Zt (ft):	12.0

- Alternative Scenario #3:  Scour and Historical Data are Available

- Channel Depth, D (ft):	4.5

- Scour Depth, Zt (ft):	12.0

- Left Historical Lateral Erosion Distance, Lh (ft)	85.0

- Right Historical Lateral Erosion Distance, Rh (ft)	85.0

- Alternative Scenario #4:  Scour, Historical Data and Cross Section Data are Available

- Cross Section ID:	LATEROS1

- Scour Depth, Zt (ft):	12.0

- Left Historical Lateral Erosion Distance, Lh (ft)	85.0

- Right Historical Lateral Erosion Distance, Rh (ft)	85.0

- FEMA Floodway Left Station	215.0

- FEMA Floodway Right Station	420.0

- Thalweg Station	355.0

# Step-by-Step Procedures

Step 1: 	Establish a New Project and Default Set-up

Step 2: 	Prepare the Cross Section Geometry

Step 3: 	Import Cross Section and Hydraulic Data

Step 4: 	Calculate Lateral Erosion Distance

(a)	Scenario #1 – No Data Available

(b)  	Scenario #2 – Scour Data Available

(c)  	Scenario #3 – Scour and Historical Data Available

(d) 	Scenario #4 – Scour, Historical and Cross Section Data Available

Step 5: 	Report and Documentation of Results

## Step 1 -	Establish a New Project and Defaults Set-up

- Click the DDMSW icon on the Desktop or Program menu to launch the DDMSW. Click OK to accept the software disclaimer as is shown in the following figure.

After the DDMSW is launched, the Select Project window is automatically opened as is shown in the following figure.

- Click the Add button on the Select Project window to start a new project (or you can start a new project by File  New Project).

- On the New Project Options form, select River Mechanics checkbox and click the OK button to close the form.

- Type “LATEROSION1” into the Reference textbox. This is the name of this newly created project. The users can choose the name as long as it does not exist in the DDMSW database.

- Type into the Title textbox a brief descriptive title of this project. (Optional)

- Type into the Location textbox the location of this project. (Optional)

- Type into the Agency textbox the agency or company name. (Optional)

- Check River Mechanics checkbox for this project.

- Type a detailed description of this project into the Comment Box under the Project Reference frame. (Optional)

- Click the Save button to save the entered data.

- Click the OK button on the Select Project window, and then click OK on the pop-up message box.  The following figure shows what the window looks like.

Note: The Project ID 00162 in the above figure is the database records unique read-only identifier of the project, which is automatically generated by the program when a new project is created. When the users create a new project, the Project ID of this new project will not be the same as the Project ID shown in the above figure.

## Step 2 – Prepare the Cross Section and Hydraulics Data

To develop the cross section and hydraulic data to be used for this project, the “LATEROS1” dataset will be used which will be imported from another project.

- From the menu bar of main application window, click River Mechanics  Import Cross Sections from Another Project to open the Import Cross Sections From Another Project form.

On the Import Cross Sections From Another Project form, select the following settings:

- Import Project Reference: 	PROJECTXSECTIONS

- Option:	Specific Cross Section

- Import Cross Section ID:	LATEROS1

Once the specified data have been selected, click the Import button.  Select Yes to proceed and hit OK to close the Import Cross Section from Another Project form.

To check if the lateral erosion cross section data has been successfully imported, open the Natural Cross Sections form (River Mechanics  Cross Section Geometry).

On the form, select “LATEROS1” for the Cross Section ID by clicking the Selector button at the right side of the ID textbox.

Compare the geometric data on the Natural Cross Sections form against the tabulated data listed below. Make necessary data edits or adjustments on the form, if necessary.  Click OK to close the form.

| Station (X) | Elevation (Y) | Notes |
| --- | --- | --- |
| 0 | 165.0 |  |
| 200 | 164.9 |  |
| 215 | 164.5 | FEMA Floodway Left Station |
| 275 | 164.3 |  |
| 318 | 164.1 | Left Bank Station |
| 320 | 163.9 |  |
| 321 | 163.85 |  |
| 325 | 163.8 |  |
| 340 | 163.6 |  |
| 345 | 160.0 |  |
| 350 | 159.9 |  |
| 355 | 159.6 | Thalweg Station |
| 365 | 159.7 |  |
| 370 | 160.0 |  |
| 375 | 161.0 |  |
| 380 | 163.0 | Right Bank Station |
| 390 | 163.2 |  |
| 420 | 164.0 | FEMA Floodway Right Station |
| 515 | 164.3 |  |
| 600 | 164.6 |  |
| 900 | 165.0 |  |

Edit the cross section geometry, if necessary, and click Save to save all changes and revisions. Click OK to close and exit the Natural Cross Section form.

To view the cross-section hydraulic data for this cross-section, open the Cross Section Hydraulics form (River Mechanics  Cross Section Hydraulics) and on the form select “LATEROS1” from the Cross Section ID listing.

On the Cross Section Hydraulics form, ensure that the Source is set to “Calculate Data”. Check to ensure that the imported data values for Flow Rate (cfs), Slope (ft/ft), and Manning’s n (Channel, LOB, and ROB) are the project values specified for this project, otherwise, edit and modify them.

The project data are provided below:

- Design Flow Rate (cfs): 	3200

- Channel Slope (ft/ft): 	0.015

- Design Manning’s n for Main Channel:	0.035

- Design Manning’s n for LOB:	0.045

- Design Manning’s n for ROB:	0.050

Click Save to save the edits, if you have made any.

To recalculate or update the hydraulic analysis on this form, click Update.

Select “This Record” from the Select Option form. Click OK to close the form.

Click Yes on the Update Cross Section Hydraulics form to continue.

Click OK to close the form.

## Step 3 -	Calculate Lateral Erosion Distance

Four (4) analysis scenarios will be presented in this tutorial. These different scenarios are dependent on availability of data for analysis. These scenarios include: (a) no data; (b) with scour data; (c) with scour and historical data; and (d) with scour, historical, and cross section data.

###### Scenario #1 - No Data

- Click on the River Mechanics  Lateral Erosion

Click Add to enter a new data and type in “LOCID” on the Location ID textbox. Make sure that all the check boxes in the Available Data frame are unchecked.

Type in “4.5” into the Channel Depth, D (ft) textbox.  This information is assumed to be the field estimate made on the channel depth.

Click Save to save the data entered.

To calculate the Lateral Erosion Distance, L (ft), click the Update button and select “This Record” from the Select Option form. Click OK to close the form.

Click Yes when the Calculate Lateral Erosion form opens.

- The following screen capture below shows the results of the analysis. The only data used was a field estimate of the Channel Depth, D (ft) at the location of interest.

###### Scenario #2 – Scour Data Available

- On the River Mechanics – Lateral Erosion form (River Mechanics  Lateral Erosion), check the Scour checkbox and leave the other two checkboxes unchecked.

Type in “12.00” into the Scour Depth, Zt (ft) textbox, while keeping the value of the Channel Depth, D (ft) at “4.50”. Click Save.

To calculate the Lateral Erosion Distance, L (ft) for the manually entered data, click the Update button and select “This Record” from the Select Option form. Click Yes to continue. Note that the estimated Lateral Erosion Distance, L (ft) is 99 ft.

As an alternative to the manual entries made, DDMSW has the capability to import the value from a scour analysis project.  To use this import feature, click the magnifying glass across the Scour Depth, Zt (ft) textbox.

On the Import Total Scour From A Project form, use the following data:

- Import Project Reference: 	BANKPROTECTIONFCD

- Import ID:	AZRIVER

- Automatically Update Scour Depth (Zt) to

- Total scour Checkbox	Check

- Click OK to close the form. [Note that the Scour Depth, Zt (ft) value has changed from “12.00” to “14.13”].

To calculate the Lateral Erosion Distance, L (ft), click the Update button and select “This Record” from the Select Option form. Click OK to close the form.

Click Yes when the Calculate Lateral Erosion form opens.

- The screen capture below shows the results of the lateral erosion analysis where scour data is available, in addition to the field estimate of Channel Depth, D (ft) at “4.50” ft.

###### Scenario #3 – Scour and Historical Data Available

- On the River Mechanics – Lateral Erosion form (River Mechanics  Lateral Erosion), make sure that the Scour and Historical Photo checkboxes are checked. Leave the Cross Section checkbox unchecked.

Type in “85.0” into the Left Historical Lateral Erosion Distance, Lh (ft) and the Right Historical Lateral Erosion distance, Lh (ft) textboxes. Leave the Channel Depth, D (ft) value at “4.50” and the Scour Depth, Zt (ft) at “14.00” unchanged. Click Save to save the data.

To calculate the Lateral Erosion Distance, L (ft), click the Update button and select “This Record” from the Select Option form. Click OK to close the form.

Click Yes when the Calculate Lateral Erosion form opens.

- The screen capture below shows the results of the lateral erosion analysis where scour and historical data are available, in addition to the field estimate of Channel Depth, D (ft) at “4.50” ft.

###### Scenario #4 – Scour, Historical and Cross Section Data Available

- On the River Mechanics – Lateral Erosion form (River Mechanics  Lateral Erosion), make sure that all the checkboxes (i.e., Scour, Historical Photo, and Cross Section) are checked.

To access the Cross Section Data for this project, click the Selector button (Magnifying glass) to the right of the Section ID textbox.  On the Select Cross Section ID form, select the “LATEROS1” from the list.

Click OK to close the Select Cross Section ID form.

- Note that by using the Cross Section Data, the “Channel Depth, D (ft)” textbox becomes “inactive” and a new value of the Channel Depth had replaced the manual data entered. This new value of the Channel Depth is the difference between the FEMA Floodway Elevation and the Thalweg Elevation (i.e., 164.50 -159.60 = 4.90 ft).

On the River Mechanics – Lateral Erosion form, click on the X Section Data button to view the Lateral Erosion Cross Section form that shows the cross section data and plot identifying the location of the banks and the FEMA Floodway limits.

Click OK to close the Lateral Erosion Cross Section form.

- To calculate the Left Lateral Erosion Distance Ll (ft), and the Right Lateral Erosion Distance, Lr (ft), click the Update button.

When the Select Option form appears, highlight “This Record” and click OK to close the form.

Click Yes when the Calculate Lateral Erosion form opens.

- The screen capture provided below shows the results of the lateral erosion analysis where scour depth, historical data, and cross-section data are available.  The summary of the results are:

- Left Lateral Erosion Distance, Ll (ft):	142.1

- Right Lateral Erosion Distance, Lr (ft): 	142.1

- Left lateral Erosion Station (ft):	72.9

- Right Lateral Erosion Station (ft):	713.8

To view the plot of the Lateral Erosion analysis results, click the Graph button. As shown on the results plot, the extent of lateral erosion on the left bank is at Station 72.9 ft and the lateral erosion on the right bank is at Station 713.8 ft.  To exit the River Mechanics Graphs form, click the OK button.

## Step 4 -	Report and Documentation of Results

In this step, the instruction will be given on how to view, print, and export the calculation results of the lateral erosion analysis.

- To view the Lateral Erosion report, click the Print… button on the River Mechanics – Lateral Erosion form.

- To print the report, click the printer symbol (       ) at the top of the form.

- To export the report to a PDF file or other file formats, click the export symbol (      ) at the top of the form.

This concludes the tutorial for the Lateral Erosion Analysis.