Drainage Design Management System for Windows

Version 6.8.0

# Tutorial # 16

# HGL Evaluation of Storm Drainage System

KVL Consultants, Inc.

HGL Evaluation of Storm Drainage System

Table of Contents

No.	Section	Page

1.0	Introduction	1

2.0	Creating a New Project	2

3.0	Set the Project Path	3

4.0	Develop the Rainfall Data	5

5.0	Develop the Sub Basin Data	8

6.0	Develop the Land Use Data	8

7.0	Update the Sub Basin Data	10

8.0	Develop the Conveyance Facilities Data	11

9.0	Build the Model Network	23

10.0	Run the Model	25

11.0	Establish Data for the StormPro Lines	26

12.0	Run StormPro	27

No.	Section	Page

HGL Evaluation of Storm Drainage System

Date Updated: May 7, 2024

Tutorial Time: 45 minutes

# 1.0	Introduction

This tutorial provides the procedure to develop and solve a storm drainage system model using DDMSW. The problem that is modeled in this tutorial is taken from the Design Example 4.6 presented in the Flood Control District’s Hydraulics Manual (2013).  The Storm Drain System Layout for the Example Problem is presented below. All the data used in DDMSW are exactly the same data from the Design Example in the Manual.

.

# 2.0	Creating a New Project

After launching DDMSW, open the Select Project form (File  Select Project) and click the ‘Add’ button to create a new project. Select Hydrology and Hydraulics and check Rational Method. Enter a unique Reference (maximum 16 characters, no spaces) name together with appropriate Title (Optional), Location (Optional), and Agency (Optional). For the purpose of this tutorial, enter the following information:

Project Reference and Other Info:

Reference	DESIGNEX46

Title  	FCDMC Hydraulics Manual Design Example 4.6

Location 	Maricopa County, Arizona

Agency 	Flood Control District of Maricopa County

Comment Box	Tutorial Project to solve Design Example 4.6 using StormPro

Project Defaults:

Model 	Rational

Land Use 	FCDMC

Rainfall 	NOAA2

Roads 	Phoenix

Inlets	Phoenix

Click ‘Save’ to save the entered data for the new project. Click ‘OK’ to proceed. Click ‘OK’ to accept the defaults.

The name of the new project is DesignEx46.

# 3.0	Set the Project Path

Open the Project Paths form (File  Project Paths) to define the Model Runs Path for the new project.

On the Project Paths form, click the ellipse (…) button on the right of the Model Runs Path textbox field.

On the Browse for folder form, navigate to the desired folder location where model run files for the project will be saved. If the desired folder location does not exist, click the ‘Make New Folder’ button to create the folder. Rename the folder (use DESIGNEX46) and select it. Click ‘OK’ to close the form.

# 4.0	Develop the Rainfall Data

Open the NOAA2 Rainfall Data form (Hydrology  Rainfall) to develop the rainfall data for the project. If a Warning dialog box shows up indicating that ‘There is no Rainfall data for the project’, click the ‘OK’ button to create a temporary Rainfall Data.

SPECIAL NOTES: For this example, NOAA2 Rainfall will be used. This is to replicate the Rainfall Data that is used in the Design Example 4.6 of the Hydraulics Manual. Normally NOAA14 rainfall would be used but to create a similar rainfall data, it is most convenient to use NOAA2.  Since the generated rainfall data are but temporary data, the data can be updated to match the data used in Design Example 4.6. In order to accomplish this, enter the following values on the Details tab of the NOAA2 Rainfall Data form:

Non Adjusted Point Rainfall (in):

2-Year, 6-Hour Rainfall 		1.20 inches

2-Year, 24-Hour Rainfall		1.44 inches

100-Year, 6-Hour Rainfall		3.25 inches

100-Year, 24-Hour Rainfall		3.95 inches

Location:

Primary Zone				7

Short duration Zone			8

Rainfall ID:

Rainfall ID				DEFAULT

Click ‘Save’ to save the data entered, then click ‘Update’ to update the tabulated Rainfall data.

Click ‘Yes’ to continue.

The updated Rainfall data generated from the new rainfall information entered are shown below:

On the NOAA2 Rainfall Data form, click ‘Graph’ to view the IDF curves.

Click ‘OK’ to close the Rainfall Intensity form. Click ‘OK’ to exit the NOAA2 Rainfall Data form.

# 5.0	Develop the Sub Basin Data

To enter the Sub Basin data, open the Sub Basins form (Hydrology  Sub Basins). Click the ‘Add’ button to start the data entry process. Enter the data for Sub Basins 0105, 0106, 0107, 0108 and 0111 as shown below. Do not click ‘Update’ yet until the land use data has been entered. If you click ‘Save’ after entering the USGE and DSGE, the Slope will be automatically calculated.

After data are entered and saved for each Sub Basin, click ‘Add’ to enter subsequent Sub Basin data. When finished, click ‘OK’ to close the Sub Basins form.

| Sub Basin | Sort | Area (acres) | Length (ft) | USGE (ft) | DSGE (ft) | Slope (ft/mi) |
| --- | --- | --- | --- | --- | --- | --- |
| 0105 | 10 | 0.47 | 390 | 100.0 | 97.0 | 40.6 |
| 0106 | 20 | 0.86 | 1000 | 100.0 | 83.0 | 89.8 |
| 0107 | 30 | 0.47 | 450 | 100.0 | 95.0 | 58.7 |
| 0108 | 40 | 0.67 | 510 | 100.0 | 97.0 | 31.1 |
| 0111 | 50 | 50.00 | 1350 | 100.0 | 90.0 | 39.1 |

# 6.0	Develop the Land Use Data

To enter the land use data, open the Land Use form (Hydrology  Land Use) then click Add. Enter the Land Use data for Sub Basins 0105, 0106, 0107, 0108 and 0111 as provided in the table below. It is necessary to modify the C values to come close to matching the values in Design Example 4.6

| Sub Basin ID | Land Use Code | Area (acres) | 2-Year Custom C | 50-Year Custom C | 100-Year Custom C |
| --- | --- | --- | --- | --- | --- |
| 0105 | 600 | 0.47 | 0.91 |  |  |
| 0106 | 600 | 0.86 | 0.91 |  |  |
| 0107 | 600 | 0.47 | 0.91 |  |  |
| 0108 | 600 | 0.67 | 0.91 |  |  |
| 0111 | 200 | 50.00 | 0.85 | .90 | .90 |

After individual Land Use data are entered into the form, click ‘Save to save the data and click ‘Add’ to enter the next Land Use record. When all the Land use data have been entered, click ‘OK’ to close and exit the Land Use form.

# 7.0	Update the Sub Basin Data

To evaluate the parameters of individual Sub Basin with consideration of the Rainfall and the Land Use data developed in the previous Sections 4 and 6, open the Sub Basins form (Hydrology  Sub Basins).   On the Details tab of the form, click the ‘Update’ button.

For Sub Basin 0111, check the Custom Tc checkbox for the 2-yr storm event.  The rationale for this customized Tc of 14.1 min is to come close to matching the data in Design Example 4.6.

Click ‘OK’ to close and exit the Sub Basins form.

# 8.0	Develop the Conveyance Facilities Data

The Storm Drain Layout in Section 1.0 shows one (1) Main Line, one (1) Lateral Line that feeds into the Main Line, and 4 Connector Pipes that feed into the Lateral Line. For this tutorial project, the StormPro backwater model is used to develop the hydraulic grade line (HGL) of the storm drain system. It is, therefore, necessary to sort the Conveyance Facilities in the correct order and establish the Line ID for each Conveyance Facility. With respect to the System Layout, Conveyance Facilities 010035 and 010040 will be Line ID 10 which outfalls into a Retention Basin. All remaining conveyance facilities (Lateral Line and Connector Pipes) with be designated as Line ID 20. Line ID 20 outfalls into Conveyance Facilities 010040.

The physical data (pipe size, pipe length, pipe invert elevations, etc.) of all the conduits / pipes in the system are tabulated below.

To enter the Conveyance Facilities data, open the Conveyance Facilities form (Hydraulics  Conveyance Facilities). Click the ‘Add’ button to start the data entry for the first record.

For the first Conveyance Facilities data, enter the following:

ID:

- Facility ID	010040

- Line ID	10

- Sort	10

Model Options:

- RP (yrs)	2

- Custom checkbox	Check

- Q (cfs)	105.2

- Model Road checkbox	Check

- First Pipe checkbox	Uncheck

- Outfall checkbox	Check

- D/S Pipe ID	Blank

Elevations:

- U/S Ground Elevation (ft)	1273.55

- D/S Ground Elevation (ft)	1270.50

- U/S Invert Elevation (ft)		1266.80

- D/S Invert Elevation (ft)		1265.50

Section Type:

- Section		Pipe

- Length (ft)		80.00

- Manning’s n		0.013

- Diameter (in)		48

- No. of Barrels		1

- Road ID		PH-MJC-XSE-80

- No. of Manholes		0

After entering all the data for the first record (Facility ID 010040), press ‘Save’.

Click ‘Add’ to enter the next record. Enter the following data for Facility ID 010035.

ID:

- Facility ID	010035

- Line ID	10

- Sort	20

Model Options:

- RP (yrs)	2

- Custom checkbox	Check

- Q (cfs)	102.0

- Model Road checkbox	Check

- First Pipe checkbox	Check

- Outfall checkbox	Uncheck

Elevations:

- U/S Ground Elevation (ft)	1274.22

- D/S Ground Elevation (ft)	1273.55

- U/S Invert Elevation (ft)		1268.30

- D/S Invert Elevation (ft)		1266.80

Section Type:

- Section		Pipe

- Length (ft)		300.00

- Manning’s n		0.013

- Diameter (in)		48

- No. of Barrels		1

- Road ID		PH-MJC-XSE-80

- No. of Manholes		0

After entering all the data for the second record (Facility ID 010035), press ‘Save’.

Click ‘Add’ to enter the next record. Enter the following data for Facility ID 010030.

ID:

- Facility ID	010030

- Line ID	20

- Sort	30

Model Options:

- RP (yrs)	2

- Custom checkbox	Check

- Q (cfs)	5.8

- Model Road checkbox	Check

- First Pipe checkbox	Uncheck

- Outfall checkbox	Check

- D/S Pipe ID	010040

Elevations:

- U/S Ground Elevation (ft)	1274.36

- D/S Ground Elevation (ft)	1273.55

- U/S Invert Elevation (ft)		1269.30

- D/S Invert Elevation (ft)		1268.80

Section Type:

- Section		Pipe

- Length (ft)		200.00

- Manning’s n		0.013

- Diameter (in)		24

- No. of Barrels		1

- Road ID		PH-MJC-XSE-80

- No. of Manholes		0

After entering all the data for the third record (Facility ID 010030), press ‘Save’.

Note: If an error appears try by modifying the Ground – D/S value until the error disappears. After that introduce the correct value.

Click ‘Add’ to enter the next record. Enter the following data for Facility ID 010025.

ID:

- Facility ID	010025

- Line ID	20

- Sort	40

Model Options:

- RP (yrs)	2

- Custom checkbox	Check

- Q (cfs)	5.8

- Model Road checkbox	Check

- First Pipe checkbox	Uncheck

- Outfall checkbox	Uncheck

Elevations:

- U/S Ground Elevation (ft)	1274.77

- D/S Ground Elevation (ft)	1274.36

- U/S Invert Elevation (ft)		1270.05

- D/S Invert Elevation (ft)		1269.80

Section Type:

- Section		Pipe

- Length (ft)		100.00

- Manning’s n		0.013

- Diameter (in)		18

- No. of Barrels		1

- Road ID		PH-MJC-XSE-80

- No. of Manholes		0

After entering all the data for the fourth record (Facility ID 010025), press ‘Save’.

Click ‘Add’ to enter the next record. Enter the following data for Facility ID 010020.

ID:

- Facility ID	010020

- Line ID	20

- Sort	50

Model Options:

- RP (yrs)	2

- Custom checkbox	Check

- Q (cfs)	5.8

- Model Road checkbox	Uncheck

- First Pipe checkbox	Uncheck

- Outfall checkbox	Uncheck

Elevations:

- U/S Ground Elevation (ft)	1274.59

- D/S Ground Elevation (ft)	1274.77

- U/S Invert Elevation (ft)		1270.24

- D/S Invert Elevation (ft)		1270.05

Section Type:

- Section		Pipe

- Length (ft)		75.00

- Manning’s n		0.013

- Diameter (in)		18

- No. of Barrels		1

- No. of Manholes		0

After entering all the data for the fifth record (Facility ID 010020), press ‘Save’.

Click ‘Add’ to enter the next record. Enter the following data for Facility ID 010015.

ID:

- Facility ID	010015

- Line ID	20

- Sort	60

Model Options:

- RP (yrs)	2

- Custom checkbox	Check

- Q (cfs)	5.8

- Model Road checkbox	Uncheck

- First Pipe checkbox	Uncheck

- Outfall checkbox	Uncheck

Elevations:

- U/S Ground Elevation (ft)	1274.44

- D/S Ground Elevation (ft)	1274.59

- U/S Invert Elevation (ft)		1270.29

- D/S Invert Elevation (ft)		1270.24

Section Type:

- Section		Pipe

- Length (ft)		20.00

- Manning’s n		0.013

- Diameter (in)		18

- No. of Barrels		1

- No. of Manholes		0

After entering all the data for the sixth record (Facility ID 010015), press ‘Save’.

Click ‘Add’ to enter the next record. Enter the following data for Facility ID 010010.

ID:

- Facility ID	010010

- Line ID	20

- Sort	70

Model Options:

- RP (yrs)	2

- Custom checkbox	Check

- Q (cfs)	4.7

- Model Road checkbox	Uncheck

- First Pipe checkbox	Uncheck

- Outfall checkbox	Uncheck

Elevations:

- U/S Ground Elevation (ft)	1274.44

- D/S Ground Elevation (ft)	1274.44

- U/S Invert Elevation (ft)		1270.31

- D/S Invert Elevation (ft)		1270.29

Section Type:

- Section		Pipe

- Length (ft)		10.00

- Manning’s n		0.013

- Diameter (in)		18

- No. of Barrels		1

- No. of Manholes		0

After entering all the data for the seventh record (Facility ID 010010), press ‘Save’.

Click ‘Add’ to enter the next and last record. Enter the following data for Facility ID 010005.

ID:

- Facility ID	010005

- Line ID	20

- Sort	80

Model Options:

- RP (yrs)	2

- Custom checkbox	Check

- Q (cfs)	3.4

- Model Road checkbox	Check

- First Pipe checkbox	Check

- Outfall checkbox	Uncheck

Elevations:

- U/S Ground Elevation (ft)	1274.96

- D/S Ground Elevation (ft)	1274.44

- U/S Invert Elevation (ft)		1270.79

- D/S Invert Elevation (ft)		1270.31

Section Type:

- Section		Pipe

- Length (ft)		190.00

- Manning’s n		0.013

- Diameter (in)		18

- No. of Barrels		1

- Road ID		PH-MJC-XSE-80

- No. of Manholes		0

After entering all the data for the last record (Facility ID 010005), press ‘Save’.

To view the list of Facilities Data entered, click the List tab.

Click ‘OK’ to close the Conveyance Facilities form now that the Conveyance Facilities Data have now been developed.

# 9.0	Build the Model Network

Prior to running the Storm Drain system created from the Section 8.0, it is essential to build the model network to define the interconnectivity of the piping / conduit system. To build the model network, open the Rational Method Network form (Hydrology  Rational Method  Network). To start, press the ‘Add’ button. Build the model network as shown below. The procedure for building a model network for Rational Method is presented in another tutorial document called, “Developing a New Project Using Rational Method”.

Once the model network is built (like the one shown above), click the ‘Check Network’ button for the program to verify if the model network that was built is complete or has issues.

# 10.0	Run the Model

To run the model, open the Run Rational Method Model form and make the following changes on the form:

- Check all the Return Periods (i.e., 2 Year, 5 Year, 10 Year, 25 Year, 50 Year, and 100 Year);

- Check the ‘Update Rational Method’ checkbox.

- Check the ‘Update Conveyance Flows’ checkbox.

Click the ‘Run Model’ button.

Click ‘OK’ to continue.

After the model is run, click ‘OK’ to close and exit the form.

# 11.0	Establish Data for the StormPro Lines

To establish the starting water surface elevation (boundary condition) for Line ID 10 (the Main Line), open the StormPro Lines form (Hydraulics  StormPro Backwater  Lines). This starting water surface elevation is used to establish the hydraulic grade line (HGL) for the entire drainage system. If left blank, the value will default to (D+Dc)/2, where D is the depth of the facility, and Dc is the critical depth.

On the StormPro Lines form, press the ‘Update’ button to start. On the Update StormPro Lines form, click ‘Yes’ to continue.

For Line ID 10, check that it is a ‘Main Line’ and enter 1269.90 for the starting water surface elevation. Do not enter any starting water surface elevation for Line ID 20 (Lateral Line) as this will be established in the Model Run.

Click ‘Update’ and ‘OK’ to close the form.

# 12.0	Run StormPro

To run the model, open the Run StormPro Model form (Hydraulics  StormPro

Backwater  Model). Select the 2 Year Return Period, check All Lines and Delete All

Prior Results checkboxes. Click ‘Save’ to save the data entered and press ‘Run Model’.

Click ‘Yes’ to continue.

Once the model run is complete, model results viewed by clicking the ‘Results’ button and can be graphed by clicking the ‘Graph’ on the StormPro Results form.

To see more detailed results, the model output file can be viewed by clicking ‘View’, and selecting ‘Output’ for the File Type.