Drainage Design Management System for Windows

Version 6.8.0

# Tutorial # 18

# StormPro Backwater Modeling

KVL Consultants, Inc.

This document contains step-by-step tutorials for the Storm Drainage Hydraulics module of DDMSW for evaluating the hydraulic grade line.

StormPro Backwater Modeling

Table of Contents

No.	Section	Page

Tutorial # 18	1

StormPro Backwater Modeling	1

1.0	Introduction	1

2.0	Set the Folder for Model Runs (File  Project Paths)	2

3.0	Modify Conveyance Facilities (Hydraulics  Conveyance Facilities)	3

4.0	Establish Line IDs (Hydraulics  StormPro Backwater  Lines)	5

5.0	Run Model (Hydraulics  StormPro Backwater  Model)	7

StormPro Backwater Modeling

Date Updated: May 7, 2024

Tutorial Time: 30 minutes

# 1.0	Introduction

This tutorial provides a working example (KVLExample7) in the use of the StormPro Backwater Model.  Prior to developing the backwater model, it was necessary to develop the hydrology for the Rational Method and to enter the data for all the conveyance facilities.  The detailed procedures for employing the Rational Method and for analyzing the Conveyance Facilities included in the network were already covered in Tutorial #3 – Developing a New Project Using Rational Method.  Tutorial #18 starts after Tutorial #3 was concluded.

The pecific requirements for running StormPro using the Pipe Network shown below include:

- Establishing a folder for the model runs

- Modifying the Conveyance Facilities

- Establish the details for the Line IDs

- Run Model

# 2.0	Set the Folder for Model Runs (File  Project Paths)

(1)	Start DDMSW

(2)	Open the Select Project form and select the List tab (File  Select Project  List tab)

(3)	From the list of Rational Method projects, select KVLExample7.

(4)	Create a copy of the project and name it as V680_KVLExample7.

(4)	With the new project selected, switch to the Details tab.  Take note on the project defaults already made and adjust the Modification Date to reflect the current Date. Click Save to save the changes entered and press OK to close the window.

(5) 	Set the Model Runs Path of the project (File  Project Paths) to C:\FCDMC\DDMSW605\Modlruns\ V680_KVLEXAMPLE7\.

# 3.0	Modify Conveyance Facilities (Hydraulics  Conveyance Facilities)

In addition to the data previously entered in Tutorial #3 (Developing a New Project Using Rational Method ) for the Conveyance Facilities, the following additional data needs to be entered:

Line ID: 	StormPro models each line separately starting with the lowest Line ID.  It is important to enter the Line ID’s in the order that the model should run.  This is to establish the starting water surface elevation for Lines entering another Line.  In the above network, all conveyance facilities in the Main Line (that goes to an Outfall) are labeled Line ID 100.  The upstream Line in this example is labeled Line ID 200.

Sort: 	For StormPro to run correctly, the Facility ID’s must be sorted in the order from Downstream to Upstream.  Use the Sort field to force the correct order.  This is critical.

Outfall: 	If a Facility ID is an outfall, then check the Outfall checkbox.  In this case, there are two outfalls.  They are Facility IDs 010105 and 010205 for Line IDs 100 and 200, respectively.

D/S Pipe ID: 	If a Facility ID enters a downstream Line, then enter the D/S Pipe ID.  In the case of Facility ID 010205 for Line ID 200, enter Pipe ID 010105 (of Line ID 100) as the D/S Pipe ID.

Manholes: 	Enter the number of manholes in each Facility ID.

Screen Captures for Facility IDs 010105, 010110 and 010205 are shown below.

Facility ID 010105:

Facility ID 010110:

Facility ID 010205:

# 4.0	Establish Line IDs (Hydraulics  StormPro Backwater  Lines)

When first going into this form, there will be no data and there will not be an Add button.  The data for the Lines are established when clicking the Update button.  In this case a warning will be given that there is no Downstream ID for Line ID 100 (because it is an Outfall).  For this Line ID 100, check Main Line.  It is important to note that if the Conveyance Facilities are modified, then the StormPro Lines should be updated before running a StormPro Model.

For a Main Line, the Starting HGL (Hydraulic Grade Line) for each return period can be entered.  If left blank, the model uses the formula (Dc + D)/2, where Dc is the critical depth and D is the height of the Facility ID.

For Lines that are not a Main Line, a Starting HGL can be entered by checking the appropriate Custom checkbox for each return period.  If left blank, the model establishes the value from the modeled Line that this Line enters.

Line ID 100:

Line ID 200:

These are the screenshots for the two Lines (Line ID 100 and Line ID 200) where the Starting HGL are automatically loaded from analysis results.

# 5.0	Run Model (Hydraulics  StormPro Backwater  Model)

On the Run StormPro Model form (Hydraulics  StormPro Backwater Model), click the Run Model button.

Options when running a StormPro Model include Return Period, Line ID and Delete Prior Results.  If All Lines check box is checked, then StormPro will model all the selected return periods for Line 100 then model all the selected return periods for Line 200 (in that order).

Click Save to accept the analysis settings entered on the form and click Run Model to execute the model.  Click Yes to continue.

After successful model run, click Results to view the analysis results.

Click Graph to view the graph of the model results.

HGL and EGL generated for Line ID 100:

To view another line and/or return period, click the View button.

Options include selecting the Line ID, Return Period, File Type and an option to graph the Energy Grade Line (Graph EGL).  When selecting a File Type the following options are available:

Results will select the data from the StormPro Results filtered for the selected Line ID and Return Period.

HGL>GE will select the data from the StormPro Results filtered for the selected Line ID, Return Period and sections where the hydraulic grade line is above the ground elevation.

Input, Output or Warning will open the model Input, Output and Warning files, respectively (See below for examples of the Input File, Output File, and Warning File).

INPUT FILE:

OUTPUT FILE:

WARNING FILE:

This concludes Tutorial # 18.