Drainage Design Management System for Windows

Version 6.8.0

# Tutorial # 21

# Pressure Flow Scour Analysis HEC-18 Procedure

KVL Consultants, Inc.

Pressure Flow Scour Analysis

[HEC-18 Procedure]

Table of Contents

No.	Section	Page

Tutorial # 21	1

Pressure Flow Scour Analysis HEC-18 Procedure	1

Pressure Flow Scour Analysis	1

[HEC-18 Procedure]	1

1.0	Introduction	1

2.0	Symbols and Definition of Terms	1

3.0	Problem Statement	1

4.0	Required Data	2

5.0	Step-by-Step Procedure	4

5.1	Step 1 - Create a New Project and Set-up Project Defaults	4

5.2	Step 2 - Prepare the Cross-Section Data and Evaluate the Section Hydraulics	6

5.2.1	Create the Bridge Cross-Section Data and Evaluate the Hydraulics	7

5.2.2	Create the Upstream Cross Section Data and Evaluate the Hydraulics	11

5.3	Step 3 - Calculate the Pressure Flow Scour	15

5.4	Step 4 - Reporting and Documentation of Results	21

# Pressure Flow Scour Analysis

# [HEC-18 Procedure]

Date Updated: May 7, 2024

Tutorial Time: 40 minutes

# Introduction

Evaluation of pressure flow scour underneath a bridge structure from significant flood events is critical for safe bridge design and for assessing bridge stability. Figure 1 below shows a typical bridge condition where pressure flow impacts the transport and stability of the bed materials giving rise to bed degradation and the formation of scour. FHWA (2012) has developed a number of useful procedures to evaluate such scour depth. The analysis procedure for predicting scour depths have been established for the following conditions:

(a) Live-bed with overtopping (or submerged) condition;

(b) Live-bed with no overtopping (or not submerged) condition;

(c) Clear-water with overtopping condition; and,

(d) Clear-water with no overtopping condition.

For a more thorough coverage on this topic, please refer to the following reference materials:

- River Mechanics Manual for DDMSW by FCDMC (April 30, 2021)

- HEC-18 – Evaluating Scour at Bridges by FHWA (April 2012).

Figure 1: Vertical contraction under the bridge (FHWA, 2012)

# Symbols and Definition of Terms

The different symbols used to describe the parameters presented in Figure 1 associated with pressure flow scour are defined below.

| No. | Symbol | Definition |
| --- | --- | --- |
| 1 | D50 | Average size of the bed material upstream of the bridge. It is the sediment particle size in which 50% are smaller (mm). |
| 2 | hb | Vertical height of the bridge opening prior to scour (ft), |
| 3 | hc | Vertical flow height in ft. |
| 4 | ht | Distance from the water surface to the lower face of the bridge girders in ft, i.e., ht = hu – hb; |
| 5 | hu, ya | Average depth of flow upstream of the bridge (ft), |
| 6 | hue | Effective upstream channel flow depth for live-bed conditions and bridge overtopping in ft. |
| 7 | hw | Weir flow height in ft. hw = ht – T for ht > T hw = 0 for ht < T; |
| 8 | Q1 | Flow in the upstream channel transporting sediment (cfs), |
| 9 | Q2 | Flow in the contracted channel section (cfs), |
| 10 | t | Maximum thickness of the flow separation zone (ft), |
| 11 | T | Height of obstruction that includes girders, deck, and parapet, ft. |
| 12 | Vc | Critical velocity (ft/s), |
| 13 | W1 | Bottom width of the upstream channel that is transporting bed material (ft). |
| 14 | W2 | Bottom width of the main channel in the contracted section less pier widths (ft). |
| 15 | y1 | Average depth of flow in the upstream main channel (ft), (hydraulic depth), |
| 16 | y2 | Average depth of flow in the contracted section (ft), (hydraulic depth), |
| 17 | ys | Pressure flow scour depth, ft. |

# Problem Statement

This tutorial document was developed to show how scour analysis is performed for bridge structures involving pressure flow conditions with live-bed and no bridge overtopping. The conditions are described below:

- All the flow is through the bridge with no overtopping.

- There are no piers (clear span)

- Upstream channel width (W1) and bridge opening width (W2) = 40 ft;

- Total discharge (Q2) = 5000 cfs;

- Upstream channel discharge (Q1) = 4000 cfs;

- Upstream floodplain discharge = 1000 cfs

- Upstream channel flow depth (y1 = hu) = 10.14 ft

- Bridge opening height (hb) = 8.0 ft;

- Deck thickness (T) = 3 ft;

- Bed material, D50 = 20 mm (Vc = 6.63 fps)

- Upstream channel velocity (V = Q1/(W1hu) = 5000/(40 x 10.14) = 9.86 fps;

Determine the magnitude of live-bed contraction scour for pressure flow conditions.

# Required Data

The following data are the basic data that will be used for the pressure flow scour analysis.

- Data at the Bridge Cross-Section

    - Bridge Cross-Section Data – The cross-section geometry where the bridge is located is provided below:

| No | Station (X) | Elevation (Y) | Notes |
| --- | --- | --- | --- |
| 1 | 0 | 20 |  |
| 2 | 20 | 19 | Left Bank Station |
| 3 | 20.01 | 4 |  |
| 4 | 40.01 | 4 |  |
| 5 | 60.01 | 4 |  |
| 6 | 60.02 | 19 | Right Bank Station |
| 7 | 80.02 | 20 |  |

- No piers (Clear Span)

- Height of obstruction/ Deck thickness, T (ft):	3.0

- Bridge opening width, W2 (ft)	40

- Bridge opening height, hb (ft):	8.00

- Flow Rates and Related Data:

- Design Flow Rate (cfs): 	5000

- Dominant Flow Rate (cfs): 	1000

- Channel Slope for Design Flow (ft/ft):	0.010

- Channel Slope for Dominant Flow (ft/ft):	0.010

- Channel Manning’s n for Design Flow:	0.035

- LOB Manning’s n for Design Flow:	0.035

- ROB Manning’s n for Design Flow:	0.035

- Channel Manning’s n for Dominant Flow:	0.035

- LOB Manning’s n for Dominant Flow:	0.035

- ROB Manning’s n for Dominant Flow:	0.035

- Data at the Upstream Cross-Section

- Upstream Cross-Section Data – The representative geometry of the upstream cross-section is provided below:

| No | Station (X) | Elevation (Y) | Notes |
| --- | --- | --- | --- |
| 1 | 0 | 20 |  |
| 2 | 20 | 19 | Left Bank Station |
| 3 | 20.01 | 4 |  |
| 4 | 40.01 | 4 |  |
| 5 | 60.01 | 4 |  |
| 6 | 60.02 | 19 | Right Bank Station |
| 7 | 80.02 | 20 |  |

- Flow Rates and Related Data:

- Design Flow Rate (cfs): 	4000

- Channel Slope (ft/ft):	0.0070

- Channel Manning’s n:	0.045

- LOB Manning’s n:	0.045

- ROB Manning’s n:	0.045

- Bottom width of upstream channel, W1 (ft):	40.00

- Sediment Data:

- Bed material, D50 (mm):	20.00

# Step-by-Step Procedure

The general procedure for evaluating scour involving pressure flows at bridges is as follows:

- Create a new river mechanics project and set-up project defaults

- Prepare the cross-section geometry and evaluate the hydraulics

- Calculate the pressure flow scour

- Reporting and documentation of results

## 5.1	Step 1 - Create a New Project and Set-up Project Defaults

- Click the DDMSW icon on the Desktop or Program menu to launch DDMSW. Click the OK button to accept the software disclaimer as shown in the following figure.

After DDMSW is launched, the Select Project window is automatically opened.

- Click the Add button on the Select Project form to start a new project (Alternatively, File  New Project).

- Select River Mechanics checkbox, then click the OK button on the New Project Options form.

- On the Select Project form, enter “V680_PRESSURE_SCOUR” into the Reference textbox. This is the name of the new project. Users can choose any project name to be entered on the Reference textbox as long as it does not already exist in the DDMSW project database.

- Type into the Title textbox a brief descriptive title of this project. (Optional) (e.g., ‘Pressure Flow Scour using HEC-18 NCHRP Procedure’).

- Type into the Location textbox the location of this project. (Optional) (e.g., ‘Maricopa County, Arizona’)

- Type into the Agency textbox the agency or company name. (Optional) (e.g., ‘Flood Control District of Maricopa County’).

- Check the River Mechanics Only checkbox.

- Type a detailed description of this project into the comment area under the Project Reference frame. (Optional) (e.g., ‘This is a tutorial project for evaluating Pressure Flow Scour using HEC-18 NCHRP Procedure’).

- Set the Modification Date using today’s date by clicking on the Calendar icon.

The following figure shows what the form should look like.

- Click the Save button to save the entered data.

Note: The Project ID “00145” in the above figure is the unique database record identifier for the project, which is automatically generated by the program. Each time a new project is created, a Project ID is assigned by the program. The Project ID assigned to your project will not necessary be the same as the Project ID shown in the above figure.

- Click the OK button to exit/close the Select Project form.

## 5.2	Step 2 - Prepare the Cross-Section Data and Evaluate the Section Hydraulics

- Two channel section data are considered in the analysis where hydraulics information would be evaluated. They are the cross-section data of: (a) upper channel reach; and, (b) bridge channel. Geometric and relevant data are entered in DDMSW to evaluate relevant flow hydraulics to be used by the program in evaluating the Pressure Flow scour depth.

- From the menu bar of the main application window, open the Cross Section Hydraulics form (River Mechanics  Cross Section Hydraulics).

### 5.2.1	Create the Bridge Cross-Section Data and Evaluate the Hydraulics

- On the Cross Section Hydraulics form, click the Add button.

- Enter “BRIDGEXS” into the Section ID textbox, then click the Save button.

- Check both the Design and the Dominant checkboxes.

- Enter “5000” into the Design Flow Rate (cfs) textbox.

- Enter “0.01” into the Design Slope (ft/ft) textbox.

- Enter “0.035” into the Design Manning’s n Channel textbox.

- Enter “0.035” into the Design Manning’s n LOB textbox.

- Enter “0.035” into the Design Manning’s n ROB textbox.

- Enter “1000” into the Dominant Flow Rate (cfs) textbox.

- Enter “0.01” into the Dominant Slope (ft/ft) textbox.

- Enter “0.035” into the Dominant Manning’s n Channel textbox.

- Enter “0.035” into the Dominant Manning’s n LOB textbox.

- Enter “0.035” into the Dominant Manning’s n ROB textbox.

- Click the Save button to save the entered data for the Bridge Section.

- Click the X Section button at the bottom of the Cross Section Hydraulics form to enter the X and Y coordinates of the Bridge Section.

- On the Natural Cross Section form, enter the first X and Y values (i.e., X= “0.00” and Y = “20.00”) from the Bridge Cross Section data table into the Station (ft) and Elevation (ft) text boxes shown below.  Click the Save & Add Record button when done to continue with the next XY dataset.

- Follow the same procedure until all the Station (X) and Elevation (Y) data pairs are entered.

The completed form should look like the figure below.

- After entering all the Station (X) and Elevation (Y) data, set the left and right overbank stations. The left overbank station is set by selecting the row with Station value of “20.00” and then clicking the “Selection” button on the right side of the Left Overbank textbox. Similarly, the right overbank station is set by selecting the row with Station value of “60.02” and then clicking the “Selection” button on the right side of the Right Overbank textbox.

- Once the data entry is complete, close the Natural Cross Section form by clicking the OK button.

- On the River Mechanics - Cross Section Hydraulics form, click the Update button to update the hydraulic analysis at the Bridge Section, BRIDGEXS, for both the Design and Dominant flow conditions.

- On the Select Option dialog box, select “This Record”, then click OK to close. When the Update Cross Section Hydraulics dialog box appears, click Yes to continue.

- After successful analysis update, the River Mechanics - Cross Section Hydraulics form should look like the figure below:

### 5.2.2	Create the Upstream Cross Section Data and Evaluate the Hydraulics

- On the River Mechanics - Cross Section Hydraulics form, click the Add button.

- Enter “UPSTREAMXS” into the Section ID textbox, then click the Save button.

- In the Entire Cross Section data frame, check the Design checkbox and uncheck the Dominant checkbox.

- Enter “4000” into the Design Flow Rate (cfs) textbox.

- Enter “0.007” into the Design Slope (ft/ft) textbox.

- Enter “0.045” into the Design Manning’s n Channel textbox.

- Enter “0.045” into the Design Manning’s n LOB textbox.

- Enter “0.045” into the Design Manning’s n ROB textbox.

- Click the Save button to save the entered data.

- Click the X Section button at the bottom of the form to enter the X and Y coordinates of the Upstream cross section.

- On the Natural Cross Section form, enter the first X and Y values (i.e., X = “0.00” and Y = “20.00”) from the Upstream Cross Section data table into the Station (ft) and Elevation (ft) text boxes as shown below.  Click the Save & Add Record button when done to continue with the next XY dataset.

- Follow the same procedure until all the Station and Elevation data pairs are entered.

- After entering all the Station and Elevation dataset, define the left and right over bank stations. The left bank station is set by selecting the row with Station value of “20” and then clicking the “Selection” button on the right side of the Left Overbank textbox. Similarly, the right bank station is set by selecting the row with Station value of “60.02” and then clicking the “Selection” button on the right side of the Right Overbank textbox.

- To close the Natural Cross Section form, click the OK button.

- On the River Mechanics - Cross Section Hydraulics form, click the Update button to update the hydraulics analysis at the upstream cross section, UPSTREAMXS.

- On the Select Option dialog box, select “This Record”, then click OK to close. When the Update Cross Section Hydraulics dialog box appears, click Yes to continue.

- The River Mechanics - Cross Section Hydraulics form should now look like the following figure:

- Click the OK button to close the form.

## 5.3	Step 3 - Calculate the Pressure Flow Scour

- From the menu bar of the main application window, open the Total Scour form (i.e., River Mechanics  Scour).

- Click the Add button to activate the necessary data entry fields

- On the Total tab of the Total Scour form, enter “PRESSQSCOUR” into the ID textbox

- Check the General check box only.

- Click the browse button  under the Method column across the General checkbox to launch Select Method dialog box.

- Select “Neill and HEC-18 With Pressure Flow” from the Select Method dialog box, then click OK to exit.

- On the Total Scour form, click the Save button to save the entered data.

- To perform the pressure flow scour analysis, click the General tab

- On the General tab of the Total Scour form, select “BRIDGEXS” for the Bridge Section ID.

- Select “UPSTREAMXS” for the Upstream Section ID.

- In the Neill Parameters (use Bridge Section) frame, select “Sand” for Exponent m textbox and select “Straight Reach” for Bend Factor Z textbox.

- In the HEC-18 Pressure Flow Upstream Parameters frame, enter “20” into the D50 (mm) textbox.

- Check the Manual Input Parameters checkbox and enter “10.14” into the Hydraulic Depth (ft) textbox, and “9.86” into the Avg Velocity (ft/s) textbox. After entering the data, uncheck the Manual Input Parameters checkbox.

- Enter “40” into the Bottom width of upstream channel, W1 (ft) textbox.

- Enter “40” into the Bottom width of contraction less piers, W2 (ft) textbox.

- Enter “8” into the Vertical size of the bridge opening, hb (ft) textbox.

- Enter “3” into the Height of the obstruction, T (ft) textbox.

- Click the Save button to save the entered data.

- Click the Update button to perform / update the analysis.

- Select “This Record” from the Select Option window, then click OK to exit.

- When the Calculate General Scour dialog box shows up, click Yes to continue.

- After the update, the general scour analysis results form should look like the screen shot shown below:

## 5.4	Step 4 - Reporting and Documentation of Results

- To view the analysis results on the screen, click the Print … button on the General tab of the Total Scour form.

- To print the results, click the printer symbol (      ).

- To export the results into a PDF file or other file formats, click the export symbol (        ) and select the preferred file format

This concludes this tutorial for Pressure Flow Scour Analysis based on HEC-18 procedure.