Drainage Design Management System for Windows

Version 6.8.0

# Tutorial # 22

# Pier Influence Zone Calculation

# HEC-18 Procedure

KVL Consultants, Inc.

Pier Influence Zone Calculation

[HEC-18 Procedure]

Table of Contents

No.	Section	Page

1.0	Introduction	1

2.0	Required Data	1

3.0	Step-by-Step Procedure	1

3.1	Step 1 - Establish a New Project and Set-up the Defaults	2

3.2	Step 2 - Prepare the Data	4

3.3	Step 3 – Calculate the Pier Influence Zone Width	6

3.4	Step 4 - Reporting and Documentation of Results	8

Pier Influence Zone Calculation

[HEC-18 Procedure]

Date Updated: May 7, 2024

Tutorial Time: 15 minutes

# Introduction

A pier influence zone is the top width (WT as shown in the figure below) of a pier local scour hole in cohesionless bed material. This tutorial computes the pier influence zone by using the procedure outlined in the Federal Highway Administration HEC-18 Manual (April 2012).

Figure 1: Top Width of Scour Hole Sketch Adapted from FHWA (2012)

# Required Data

For this tutorial on calculating pier influence zone, use the following conditions:

- Pier Scour Depth, Ys (ft):	15.00

- Ratio of bottom width to depth of local pier scour, B:	1.0

- Angle of repose of the bed material in water, θ (degrees):	44.00

- Distance from outside edge of pier, X (ft):	10.00

# Step-by-Step Procedure

The step-by-step procedure for calculating pier influence zone is described as follows:

Step 1: 	Establish a New Project and Set-up the Defaults

Step 2: 	Prepare the Data

Step 3: 	Calculate the Pier Influence Zone Width

Step 4:	Reporting and Documentation of Results

## Step 1 - Establish a New Project and Set-up the Defaults

- Click the DDMSW icon on the Desktop or Program menu to launch the DDMSW. Click the OK button to accept the software disclaimer as shown in the following figure.

After the DDMSW is launched, the Select Project form is automatically opened as shown in the following figure.

- Click the Add button on the Select Project form to start a new project (Alternatively, File  New Project  Add).

- On the New Project Options dialog box, select the River Mechanics checkbox, then click the OK button to close / exit the form.

- On the Select Project form (Details tab), enter “V605_PIER_INFLUENCE” into the Reference textbox. This is the name of the new project. Users can choose any name for the Reference textbox as long as it does not exist already in the current DDMSW project database.

- Type into the Title textbox a brief descriptive title for this project. (Optional) (e.g., ‘Pier Influence Zone Calculation using HEC-18 Procedure’).

- Type into the Location textbox the location of this project. (Optional) (e.g., ‘Maricopa County, Arizona’)

- Type into the Agency textbox the agency or company name. (Optional) (e.g., ‘Flood Control District of Maricopa County’).

- Check the River Mechanics Only checkbox.

- Type a detailed description of this project into the comment area under the Project Reference frame. (Optional) (e.g., ‘This is a tutorial project for calculating Pier Influence Zone Width using HEC-18 Procedure’).

- Set the Modification Date using today’s date by clicking on the Calendar icon.

- Click the Save button to save the entered data. The following figure shows what the current form looks like.

- Click the OK button to exit the Select Project form.

Note: 	The Project ID “00146” in the above figure is the unique database record identifier for the project, which is automatically generated by the program.  Each time a new project is created, a Project ID is assigned by the program. The Project ID assigned to your project will not necessary be the same as the Project ID shown in the above figure.

## Step 2 - Prepare the Data

- From the menu bar of the main application window, click River Mechanics  Scour, to open the Total Scour form.

- Click the Add button to activate the necessary data entry fields.

- On the Total Scour form (Total tab), enter “PIERINF01” into the ID textbox.

- Check the Pier Influence checkbox only.

- Click the Save button to save the entered data. The Total Scour form should look like following figure.

## Step 3 – Calculate the Pier Influence Zone Width

- On the Total Scour form, select the Pier Influence Tab

- Enter “15” into the Pier Scour Depth, Ys (ft) textbox.

- Enter “1.0” into the Ratio of bottom width to depth of local pier scour, B textbox.

- Enter “44” into the Angle of repose of the bed material in water, (degrees) textbox.

- Enter “10” into the Distance from outside edge of pier, X (ft) textbox.

- Click the Save button to save the entered data.

- Click the Update button to update or perform the analysis.

- Select “This Record” from the Selection Option dialog box, then click Yes on the Calculate Pier Influence Scour dialog box to continue.

- After the update, the form should look like the figure below.

## Step 4 - Reporting and Documentation of Results

- To view the results on the screen, click the Print … button on the Local tab of the Total Scour – MB: 01 – ID: PIERINF01 form. A report will be generated as shown.

- To print a hard copy of the results, click the printer symbol (      ).

- To export the results to a PDF file or to other file formats, click the export symbol (          )

This concludes the tutorial for pier influence zone calculation.